# Space-Based AI Data Centers: Physics, Mathematics, Simulation, and Economics

Samarjith Biswas, PhD

Table of Contents

# Space-Based AI Data Centers

# Physics, Mathematics, Simulation, and Economics of an Orbital Compute Constellation

### An engineering analysis of a space-based AI compute constellation

Author: Samarjith Biswas, PhD Date: June 2026 Reference architecture: Google Project Suncatcher (arXiv:2511.19468) Companion code: github.com/Samarjithbiswas/space-based-ai-datacenter (20-module model, 118 tests)

### Abstract

Space-based artificial-intelligence computing moved from concept to flight demonstration in 2025-2026. This document derives, from first principles, the complete physical and economic model of an orbital AI data center, implements every model in open and validated code, and reports the results, sensitivities, and forecasts. It spans seventeen subsystems, orbital mechanics, the neutral atmosphere and orbital decay, the ionizing-radiation and orbital-debris environments, thermal control, electrical power, structures, attitude control and optical pointing, inter-satellite and ground communications, propulsion and disposal, reliability, compute and workload, and techno-economics, and closes them into one self-consistent design point and constellation. Each chapter presents the governing equations with their derivations and worked numerical examples, the simulation that produces the results, and a discussion of what the result means. The central conclusions are that radiative cooling is the solved part of the problem and is now flight-proven; that the binding, unsolved constraints are orbital-debris survivability in a tight formation, credible end-of-life disposal, and economics; and that the launch-cost parity which underwrites the business case is later and less certain than commonly advertised. Every load-bearing fact is cited to a primary source, every forward-looking number is flagged as a projection, and every figure and number is generated by the accompanying open-source model.

## Table of Contents

Front matter, Abstract · Nomenclature · Physical constants

Part I, Introduction and context 1. Introduction and motivation 2. The state of the art (2025-2026) 3. Reference architecture and scope 3A. Design methodology, margins, and readiness

Part II, The orbital environment 4. Orbital mechanics of a circular orbit 5. Sun-synchronous orbits and J2 precession 6. Eclipse geometry and the dawn-dusk advantage 7. The neutral atmosphere and orbital decay 8. The ionizing-radiation environment 9. The orbital-debris environment

Part III, Spacecraft subsystems 10. Thermal control 11. Electrical power 12. Structures and mass 13. Attitude determination and control; optical pointing 14. Communications: inter-satellite and ground 15. Propulsion and end-of-life disposal 16. Reliability and constellation availability 17. Compute and workload

Part IV, Integration, simulation, and results 18. The integrated system model 19. Uncertainty quantification and sensitivity 20. Results: reference design point and constellation

Part V, Economics and predictions 21. Techno-economics: cost, NPV, and the levelized cost of compute 22. The launch-cost-parity prediction 23. Capability and risk trajectory, 2027-2035

Part VI, Synthesis 24. Caveats and limitations 25. Conclusions and the recommended demonstrator

Part VII, Detailed results, data, and worked computations 26. A worked end-to-end computation 27. Detailed results tables (T1-T13) 28. Constellation design and relative dynamics 29. Comparison with terrestrial data centers

Back matter, References · Appendix A: master equation list · Appendix B: nomenclature · Appendix C: data sources and confidence · Appendix D: reproduction and code

## Nomenclature

| Symbol | Meaning | Units |
| --- | --- | --- |
| μ | Earth gravitational parameter | m3 s−2 |
| RE | Earth mean radius | m |
| J2 | Earth oblateness coefficient | – |
| a, h, r | semi-major axis, altitude, orbital radius | m |
| v, n, T | speed, mean motion, period | m/s, rad/s, s |
| E | specific orbital energy | J/kg |
| ρ | neutral atmospheric density | kg/m3 |
| CD, A/m | drag coefficient, area-to-mass ratio | –, m2/kg |
| σ, ε, αs | Stefan-Boltzmann constant, emissivity, solar absorptivity | W/m2K4, –, – |
| T, Q, Rth | temperature, heat load, thermal resistance | K, W, K/W |
| S0, F, aalb | solar constant, view factor, Bond albedo | W/m2, –, – |
| Φ, Nf, rt | debris flux, fragment count, target radius | m−2yr−1, –, m |
| Δv, Isp | velocity increment, specific impulse | m/s, s |
| G, Lfs, λ, D | telescope gain, free-space loss, wavelength, aperture | dB, dB, m, m |
| MTBF, R(t) | mean time between failures, reliability | h, – |
| η, MFU, PUE | efficiency, model-FLOPs utilization, power-usage effectiveness | – |
| r, CRF, NPV | discount rate, capital-recovery factor, net present value | –, –, USD |

## Physical Constants

| Symbol | Value |
| --- | --- |
| μ | 3.986004418×1014 m3s−2 |
| RE | 6.371×106 m |
| J2 | 1.08263×10−3 |
| σ | 5.670374×10−8 W m−2K−4 |
| hP, kB, c | 6.62607×10−34 J s, 1.38065×10−23 J/K, 2.99792458×108 m/s |
| g0 | 9.80665 m/s2 |
| S0 | 1361 W/m2 |
| TCMB | 2.725 K |
| aalb, qIR | 0.30, 237 W/m2 |

# PART I, INTRODUCTION AND CONTEXT

## 1. Introduction and Motivation

Abstract. This chapter sets out the case for examining artificial-intelligence data centers in orbit: the energy and thermal pressure on terrestrial infrastructure, the three real physical advantages of orbit, and the severe penalties that come with them. It states the study’s purpose, a complete first-principles model with an honest separation of demonstrated capability from projection, and how the book is organized.

The energy and thermal demands of artificial-intelligence computing have grown faster than terrestrial infrastructure can comfortably absorb. Training and inference clusters are now provisioned in tens to hundreds of megawatts; cooling, water, land, and grid interconnection have become first-order constraints. Against this backdrop, placing computation in orbit is no longer purely speculative. Orbit offers three physical advantages that are real and quantifiable: a continuous, high-flux solar resource in the right orbit; an effectively unlimited radiative heat sink at the 2.725 K cosmic-microwave-background temperature; and freedom from terrestrial land and water constraints. It also imposes severe penalties: a harsh radiation environment, a congested and debris-laden orbital neighborhood, the impossibility of hands-on repair, the cost of launch, and a regulatory obligation to dispose of the hardware responsibly.

The purpose of this analysis is neither advocacy nor dismissal. It is to build the complete engineering model of an orbital AI data center from first principles, to compute the numbers carefully, and to state plainly which problems are solved and which are not. The first report in this series (November 2025) examined a single question, whether passive radiative cooling can keep a processor within its junction-temperature limit in low Earth orbit, and answered yes. This complete edition addresses the entire system: not only whether the chip stays cool, but whether the spacecraft survives, communicates, points, powers itself, disposes of itself, and pays for itself.

The work is organized as a book so that each result can be traced from physical principle, through derivation and code, to a number with an uncertainty band and a citation. Where the analysis corrects an earlier assumption or contradicts a popular claim, it says so. Where a number is a vendor projection rather than a demonstrated capability, it is flagged. The accompanying software is open, validated by 122 automated tests, and reproduces every figure and value in this volume with a single command.

From the three advantages to governing relations. The qualitative case made above rests on three physical quantities that can be written down explicitly: the solar power available to a collector, the heat that a radiator can reject to deep space, and the balance between the two that fixes the operating temperature of the spacecraft. Stating the relations now gives the scaffolding on which the later system chapters are built.

Available solar power. A flat collector of area Ac and conversion efficiency η whose normal is tilted by an angle θ from the solar direction intercepts an electrical power

Pelec=η S Accosθ,

where S is the solar irradiance. The cosine factor is the projection of the collector area onto the plane perpendicular to the incoming rays; when the panel points directly at the Sun, θ=0 and cosθ=1, recovering the maximum Pelec=ηSAc. The continuity of the orbit’s solar resource is the statement that a suitable orbit keeps S near its full value and θ near zero through most of each revolution.

Radiative rejection to deep space. Heat leaves the spacecraft almost entirely by thermal radiation, since there is no surrounding fluid to carry it away by convection. A radiator of area Ar, emissivity ε, and surface temperature Tr viewing a background at temperature Tb rejects a net power given by the Stefan-Boltzmann law,

Qrad=ε σ ArTr4−Tb4,

with σ the Stefan-Boltzmann constant. The fourth-power dependence is the central fact of orbital thermal design; doubling the radiator temperature raises its rejection by a factor of sixteen, so a modest rise in allowable surface temperature buys a large reduction in radiator area.

Why the cold background is almost free. The background term carries the cosmic-microwave-background temperature Tb=2.725 K introduced earlier. Because Tb enters only as Tb4, and because any practical radiator runs hundreds of kelvin above it, the background contributes negligibly to the rejected power. Writing the bracket in (1.2) as a fractional correction,

QradεσArTr4=1−TbTr4,

shows the size of the correction at a glance. For a radiator at Tr=300 K the ratio Tb/Tr is about 9.1×10−3, so (Tb/Tr)4≈7×10−9; the cold sink is, for engineering purposes, a perfect 0 K reservoir.

Steady-state energy balance. In thermal equilibrium the heat dissipated by the computing payload, Qdiss, must be carried off by the radiator, so equating Qdiss to (1.2) and solving for the area gives the first sizing relation of the study,

Ar=Qdissε σTr4−Tb4.

The design implication is direct: radiator area scales inversely with the fourth power of the achievable surface temperature, so the entire feasibility of orbital cooling turns on running the radiator as warm as the processor’s junction limit and the heat-transport path will allow.

## 2. The State of the Art (2025-2026)

Abstract. This chapter surveys what has actually flown by 2026, the single accelerator in orbit, the early data-center nodes, and the published constellation reference design, and distinguishes it from what the vision still requires. It also flags the load-bearing role, and the limits, of a non-peer-reviewed vendor preprint.

Three programs anchor the field as of mid-2026. Their status here is drawn from primary sources.

Google Project Suncatcher. The reference architecture for this study. Google’s research paper (arXiv:2511.19468) describes an illustrative constellation of eighty-one satellites arranged in a cluster of approximately one-kilometer radius, in a dawn-dusk sun-synchronous low-Earth orbit at a mean altitude of 650 km, with the satellites separated by roughly 100 to 200 m and connected by free-space optical links. Each satellite carries Google Tensor Processing Units. Google and Planet Labs have announced a learning mission to launch two prototype satellites carrying TPUs by early 2027; Planet will build and operate the platform. These facts are corroborated by Google’s and Planet’s own releases. The “by early 2027” date is a corporate target subject to slip.

Starcloud. In a significant demonstration, Starcloud flew an NVIDIA H100 GPU, the highest-power data-center-class processor flown to date, aboard the approximately sixty-kilogram Starcloud-1 satellite, launched on 2 November 2025. NVIDIA reports the satellite carries an H100 with 80 GB of high-bandwidth memory and on the order of four petaflops of AI compute. Starcloud has articulated a long-term vision of a five-gigawatt orbital data center with solar and cooling panels spanning roughly four kilometers on a side. The five-gigawatt figure is an aspirational early-2030s engineering target, not a demonstrated capability. Notably, Starcloud’s marketing claim of “ten times cheaper including launch” failed independent verification and should be treated with caution.

Axiom Space. Axiom deployed its first Orbital Data Center nodes to low Earth orbit on 11 January 2026 and has stated a goal of scaling on-orbit processing from kilowatts to megawatts on future nodes. Megawatt scale is a future goal, not current capacity; a claim that the first nodes would launch by the end of 2025 was inaccurate (actual deployment was January 2026).

The state of the art shows a clear gap between what has been demonstrated, one data-center-class GPU in orbit, ground-based radiation testing of a TPU, a bench-scale 1.6-terabit optical link, and initial kilowatt-class nodes, and what the vision requires, megawatt-to-gigawatt clusters, flown multi-terabit formation links, dramatically cheaper launch, and validated formation station-keeping. None of the latter has been demonstrated.

From the demonstrated single accelerator to a cluster, a scaling relation. The chapter contrasts one data-center-class GPU in orbit with a vision that requires megawatt-to-gigawatt clusters. To make that gap quantitative, let a cluster aggregate N identical accelerators, each drawing electrical power Pnode. The total electrical demand is simply additive,

Pcluster=N Pnode.

The flown Starcloud-1 carried a single H100, so N=1. Taking the H100 thermal envelope of order 0.7 kW as Pnode, the number of such nodes implied by an aspirational five-gigawatt target follows by inverting (2.1),

N=PclusterPnode=5×109 W0.7×103 W≈7×106.

The seven-million-node figure that this estimate yields is not a demonstrated capability; it is a way of reading the five-gigawatt target as a node count, and it makes plain why the chapter treats that target as a long-horizon engineering aspiration rather than a near-term deliverable. The design implication is that closing the gap is a problem of integration at scale, not of a single more capable processor.

Power supplied sets the achievable compute. The same accounting can be turned around to express the useful output of a node. If η is the fraction of supplied electrical power that reaches the accelerator after conversion and distribution losses, and Eop is the energy cost of one arithmetic operation, then the sustained throughput is

Rops=η PnodeEop.

For a fixed device Eop is set by the silicon, so on orbit the throughput is bounded by the power that the platform can deliver and the heat it can reject, not by the processor alone.

Formation links scale with the square of cluster count. The reference architecture connects roughly eighty-one satellites by free-space optical links. If every satellite must maintain a line of sight to every other satellite in a fully connected cluster of Nsat members, the number of distinct links is the number of unordered pairs,

L=Nsat2=Nsat(Nsat−1)2.

For the illustrative Nsat=81 this gives L=81⋅80/2=3240 pairwise links. Practical constellations route over a sparser subset, yet (2.4) shows why link budgeting, pointing, and station-keeping dominate the formation problem as the cluster grows; the demonstrated bench-scale optical link is one element of a connectivity requirement that grows quadratically with the number of satellites.

## 2A. The Original Feasibility Study: Passive Cooling of a TPU Node

Abstract. This chapter reproduces the original thermal-feasibility study in full: a 1450-watt node held within its junction-temperature limit at 650 kilometers by passive radiative cooling, with a margin of about fourteen degrees. It records the model, the worked numbers, the single-fault case, and the caveats that motivated the complete analysis that follows.

Sources: radiative and conductive relations from Incropera and DeWitt [21]; spacecraft thermal practice and white-paint properties from the Gilmore handbook [20]. Reproduced by orbital_dc.thermal.report_i_baseline().

The work that opened this line of analysis asked a single, narrow question: can a node carrying several high-power accelerators be held within its temperature limit at 650 kilometers using only passive radiative cooling, with no refrigeration and no consumable coolant. The answer governs everything that follows, because if the thermal case fails at the node level there is nothing to scale. This chapter reproduces that original steady-state design point in full, since the rest of the book extends and stress-tests it; the general thermal theory it draws on is developed in Chapter 10.

The heat load. The reference node dissipates four accelerators at a per-chip power of 300 watts, together with avionics and a parasitic allowance:

Qtotal=4×300+150avionics+100parasitic=1450 W.

Radiator temperature. All of this heat leaves as thermal radiation. For a gray emitter of area A and emissivity ε radiating from ns sides into a cold background, the steady-state panel temperature follows from the Stefan-Boltzmann balance,

Qtotal=ε σ A ns Trad4 ⇒ Trad=Qtotalε σ A ns1/4.

With two two-square-meter panels treated as a single-sided four-square-meter emitter, Z-93 white paint at its end-of-life emissivity of 0.85, the panel settles at Trad=21.3 ∘C. The radiator is comfortable; the difficulty is entirely in getting the heat from the silicon to it.

The conduction chain. Between the transistor junction and the panel sits a series of thermal resistances: junction to case, the thermal interface material, the cold-plate base, the heat pipe, and the radiator interface. In series they add,

Rth=rjc+rtim+rbase+rpipe+rrad.

The original budget is 0.150+0.020+0.060+0.050+0.020=0.300 K/W after a modest interface and heat-pipe improvement, down from 0.350 K/W before it. The junction-to-case term dominates and is fixed by the package; the terms a designer can influence are the interface material and the heat pipe.

Junction temperature and the margin. The junction sits above the radiator by the product of per-chip power and chain resistance,

Tj=Trad+Pchip Rth=21.3+300×0.300=111.3 ∘C.

Against a junction limit of 125 ∘C, this leaves a margin of 13.7 ∘C. The case closes, but not by much. The single result that named this study is that margin: passive cooling of a 300-watt-class accelerator at this altitude is feasible, and feasible only with a thin allowance.

Robustness. A spacecraft thermal path carries redundant heat pipes, so the design must survive one failing. If a single pipe drops out, the pipe resistance rises by about one part in seven, adding 0.011 K/W to the chain,

Rfail=Rth+Δrpipe=0.300+0.0114=0.3114 K/W,  Tj,fail=Trad+Pchip Rfail=114.8 ∘C.

The junction climbs to 114.8 ∘C, so the margin narrows to about ten degrees but stays positive. The single-fault case survives.

What the study did and did not establish. Three caveats travel with this result, and they set the agenda for the rest of the book. First, the per-chip power was taken as 300 watts; the actual figure for the reference accelerator is undisclosed and is more plausibly in the 150-to-200-watt range, in which case the junction sits well below the limit and the margin is larger, not smaller. The 300-watt case is therefore conservative. Second, and pointing the other way, the analysis is a single-node steady state; it does not by itself address the sharper wall that appears as per-chip power rises, where above roughly 400 watts the interface term alone overruns the entire budget and a passive chain can no longer cope, a point developed in Chapter 10. Third, the original study was candid that the thermal case being feasible does not make the system economical: its own break-even analysis was negative, which is exactly the question Chapter 22 takes up in detail. The thin margin and the per-chip ceiling are what motivated the full survivability, integration, and economic treatment that makes up the remainder of this book.

Deriving the radiator temperature step by step. The Stefan-Boltzmann balance in Eq. (2A.2) deserves to be unpacked, because every later thermal argument rests on it. A gray surface at uniform temperature Trad emits a hemispherical flux ε σ Trad4 per unit area. In orbit the panel also receives radiation from its surroundings, so the net emitted flux is the difference between what it sends out and what it absorbs from an effective sink at temperature Tsink,

qnet=ε σTrad4−Tsink4.

Multiplying by the participating area A ns and equating to the dissipated load gives Qtotal=ε σ A ns (Trad4−Tsink4). Eq. (2A.2) is the limiting form in which Tsink4≪Trad4, which is the cold-background assumption used for the baseline. Isolating the temperature returns the quartic root,

Trad= Tsink4+Qtotalε σ A ns1/4.

The quartic dependence is the central fact of passive cooling: the radiator temperature responds only to the fourth root of the load, so doubling Qtotal raises Trad by a factor of just 21/4≈1.19 in absolute terms, while every kelvin gained here is subtracted directly from the junction margin.

Sensitivity of the radiator to a warm sink. Because the baseline neglects Tsink, it is useful to bound the penalty of a non-zero sink. Differentiating Eq. (2A.7) at fixed load shows how strongly the panel tracks its environment,

∂Trad∂Tsink=TsinkTrad3.

A cold sink leaves the baseline essentially unchanged, while a sink approaching the panel temperature drives the sensitivity toward unity, at which point passive rejection collapses.

A worked sizing inversion. The same balance fixes the area a designer must install. Rearranging Eq. (2A.2) for the cold-background case and inserting the chapter’s reference values, Qtotal=1450 W, ε=0.85, and the settled panel temperature Trad=21.3 ∘C=294.45 K,

A ns=Qtotalε σ Trad4=14500.85×5.67×10−8×(294.45)4 m2≈4 m2,

which recovers the four-square-meter single-sided emitter of the baseline and confirms the stated operating point. The design implication is that radiator area is set by the fourth power of the chosen panel temperature, so the cheapest lever for margin is a colder panel through higher emissivity or more area, not a lower junction-to-case resistance, which the package fixes.

## 3. Reference Architecture and Scope

Abstract. This chapter fixes the reference architecture used throughout the book, an eighty-one-satellite, tight-formation constellation in a dawn-dusk sun-synchronous orbit, and states the scope and boundaries of the analysis.

Top view of the reference cluster: about eighty satellites within a one-kilometre circle, linked by laser at roughly one hundred and fifty metre spacing.

Throughout this analysis the reference design follows the Suncatcher parameters: a dawn-dusk sun-synchronous orbit at 650 km, an eighty-one-satellite cluster of one-kilometer radius, and inter-satellite spacing of roughly 150 m. The reference satellite bus is taken to carry four accelerators with an avionics and support complement, consistent with the mass budget developed in Chapter 12. Where the analysis examines higher-power chips (NVIDIA H100 at 700 W, B200 at 1000 W, the GB200 GPU at 1200 W), it does so to test the scaling of each subsystem, because the trend in AI accelerators is toward higher, not lower, per-chip power.

The scope of the models is conceptual to preliminary design (Phase A in aerospace terms). The intent is to size each subsystem, rank the risks, and quantify the trades, not to produce a certifiable flight design. Several domains are treated at first order with explicit uncertainty bands, and a number of analyses that a flight program would require (a thermal finite-element model, a closed-loop attitude-control simulation, formal failure-modes analysis, and runs of the official agency debris and radiation tools) are out of scope and are identified as such in Chapter 24. Stating the fidelity plainly is essential: the value of the model lies in its breadth and its transparency, not in a claim of flight accuracy.

Cluster geometry and inter-satellite spacing. The reference cluster places N satellites within a disc of radius R, and the chosen figures are N=81 and R=1 km. If the satellites are distributed so that each occupies an equal share of the disc area, the area per satellite is the total area divided by the count,

Asat=πR2N.

A convenient single length scale for a uniform planar arrangement is the side of the square that contains this area, which gives the characteristic spacing

s = Asat = R πN.

Substituting the reference values, s=(1000 m)π/81=(1000 m) (1.772/9)≈197 m, which is the same order as the quoted nearest-neighbour spacing of roughly 150 m. The uniform area-per-satellite estimate is an upper bound on the typical separation; the quoted 150 m reflects a denser central arrangement than a uniform fill, since the satellites are clustered rather than spread evenly over the disc. The design implication is that station-keeping and collision-avoidance budgets must hold relative position to a small fraction of s, so the control authority required scales inversely with the spacing the architecture selects.

Orbital period and ground-track repetition. At the reference altitude h=650 km the orbital radius is a=R⊕+h, and the Keplerian period follows from equating gravitational and centripetal acceleration, μ/a2=ω2a with ω=2π/T, which rearranges to

T=2π a3μ,

where μ is the Earth gravitational parameter. The dawn-dusk sun-synchronous condition then fixes the orbital plane precession rate Ω to match the mean motion of the Earth about the Sun, Ω=2π/(1 yr), through the nodal-regression relation

Ω=−32 J2R⊕a2 n cosi,

with n=2π/T the mean motion, J2 the dominant geopotential coefficient, and i the inclination. Equation (3.4) shows that for a prescribed altitude only one inclination, slightly retrograde, satisfies the sun-synchronous constraint. The design implication is that altitude and inclination are not independent choices: once h is set for the cluster, the dawn-dusk illumination that keeps the radiators in shadow and the arrays in near-continuous sunlight dictates i, and any later altitude trade reopens the attitude and power geometry together.

## 3A. Design Methodology, Margins, and Readiness

Abstract. This chapter sets the design discipline: the mass-growth and power allowances drawn from established flight practice, and an explicit technology-readiness assessment that locates each capability on the nine-level scale.

Sources: margin and readiness practice from the NASA Systems Engineering Handbook [13], AIAA S-120 [14], the Goddard concurrent-design practice [15], and the SMAD systems text [18].

A conceptual design is judged not only by its point estimates but by the allowances it carries against growth and uncertainty. This analysis follows the practice of the NASA Systems Engineering Handbook (NASA/SP-2016-6105), which tracks resource margins as technical performance measures rather than prescribing fixed numbers, together with the quantitative allowances that flight programs apply.

Mass growth. AIAA S-120-2006 sets a mass-growth allowance by hardware category for an estimated design: 30 percent for small electronics and solar arrays, 25 percent for structures, mechanisms, thermal control, batteries, and propulsion, 20 percent for large electronics, and 60 percent for harness. On top of the per-item allowance, the NASA Goddard Integrated Design Center applies a system-level mass margin of about 30 percent at the pre-Phase-A concept stage. Published mass-growth data from operational programs show actual growth often exceeds the AIAA allowance, so these figures are treated as lower bounds. The reference dry mass reported in Chapter 12 is a current-best-estimate; applying the allowances above (implemented in the margins module) gives the predicted mass a program would budget.

Power. The Goddard practice sizes the power system for a minimum 30 percent contingency of delivered power over the current-best-estimate load, evaluated at the worst-case operational scenario at end of life. The dawn-dusk power margin of Chapter 11 is interpreted against this rule.

Readiness. Progress is measured on the NASA nine-level technology-readiness scale, from basic principles observed (level 1) to a system flight-proven through successful operations (level 9). The components of the reference architecture sit near the middle: a single accelerator has operated in orbit and a processor has been ground-tested for radiation (component levels 5 to 6), while the constellation-scale system, the flown multi-terabit formation links, and validated formation station-keeping remain at low system readiness. The gap between component and system readiness is the central programmatic risk identified in Chapter 23. Assessing each major capability against the nine-level scale makes that gap explicit.

| Capability | Demonstrated status, 2026 | Assessed TRL | Gating step to the next level |
| --- | --- | --- | --- |
| Data-center-class accelerator in orbit | one GPU flown (Starcloud-1, Nov 2025) | 6 | sustained multi-unit operation on orbit |
| Processor radiation tolerance | ground-tested, no hard failures to 15 krad | 5 | in-flight dose and upset-rate confirmation |
| Node-level thermal control | kilowatt-class pumped-loop nodes flown | 5 | a megawatt-class radiator flown |
| Multi-terabit formation optical link | bench-scale link only | 4 | a flown link between two satellites |
| Formation station-keeping at sub-km spacing | not demonstrated | 3 | two-satellite close-formation flight |
| Autonomous cluster debris avoidance | not demonstrated | 2 | a closed-loop avoidance maneuver on orbit |
| Propulsive disposal within five years | components flight-proven, configuration unproven | 5 | an end-to-end de-orbit demonstration |
| Megawatt-to-gigawatt cluster | conceptual | 2 | a subscale multi-satellite cluster |

The assessment confirms the central pattern: the building blocks sit at component levels 4 to 6, while every capability that depends on the constellation operating as a coordinated system sits at levels 2 to 3. The program risk is concentrated in the system-integration steps, not the components.

| Allowance | Value | Source |
| --- | --- | --- |
| Mass growth, small electronics and solar array | 30 percent | AIAA S-120-2006 |
| Mass growth, structures, thermal, battery, propulsion | 25 percent | AIAA S-120-2006 |
| Mass growth, large electronics | 20 percent | AIAA S-120-2006 |
| Mass growth, harness | 60 percent | AIAA S-120-2006 |
| System-level mass margin, pre-Phase-A | 30 percent | NASA GSFC Integrated Design Center |
| Power contingency over best estimate, worst-case end of life | 30 percent minimum | NASA GSFC Integrated Design Center |
| Readiness framework | levels 1 to 9 | NASA Systems Engineering Handbook, NASA/SP-2016-6105 |

Composing the mass allowances. The allowances quoted above act multiplicatively on a current-best-estimate, not additively across the system. Let mi be the current-best-estimate mass of hardware item i and let gi be the AIAA S-120 growth fraction for that item’s category. The predicted, or maximum-expected, mass of the item is

mi pred=mi (1+gi).

Summing over the parts list gives the predicted dry mass before any system-level reserve,

Mdry pred=i​mi (1+gi).

The system-level mass margin of the Goddard practice is then applied on top of the aggregated prediction, so the mass a program would budget is

Mbudget=(1+Msys)i​mi (1+gi),

with Msys=0.30 at the pre-Phase-A stage. It is convenient to express the combined effect as an effective system growth fraction relative to the raw estimate M0=i​mi,

Geff=MbudgetM0−1=(1+Msys) i​mi (1+gi)i​mi−1,

in which the fraction is the mass-weighted average of (1+gi). Because the per-item term and the system reserve compound, Geff exceeds the simple sum of the two contributions.

Worked example. Consider a notional item set whose current-best-estimate mass is dominated by structure carrying g=0.25, so the mass-weighted average of (1+gi) is close to 1.25. Applying the system reserve of (3A.3),

Geff≈(1.30)(1.25)−1=0.625,

an effective allowance near 62 percent over the raw estimate, well above the 30 percent system reserve taken alone. This is the figure a designer should hold in mind when reserving launch capacity.

Power contingency at end of life. The power rule is a worst-case inequality rather than a point sizing. Writing PgenEOL for the array power delivered at end of life in the worst-case operational geometry, and Pload for the current-best-estimate load, the contingency requirement is

PgenEOL−PloadPload≥CP,  CP=0.30.

Equivalently the array must be sized so that PgenEOL≥(1+CP) Pload. Since the array degrades over the mission, the beginning-of-life requirement is larger by the inverse of the survival fraction ηdeg=PgenEOL/PgenBOL,

PgenBOL≥(1+CP) Ploadηdeg.

The design implication is that contingency and degradation stack: the array must be oversized at deployment by both the 30 percent reserve and the reciprocal of its end-of-life survival fraction, and the dawn-dusk margin of Chapter 11 should be checked against the worst-case end-of-life form of (3A.6), not against a beginning-of-life value.

# PART II, THE ORBITAL ENVIRONMENT

## 4. Orbital Mechanics of a Circular Orbit

Abstract. This chapter derives the small set of circular-orbit quantities, speed, period, and specific energy, on which every later chapter depends, and works the reference orbit as an example.

Geometry of a Keplerian orbit: Earth at one focus, the major axis 2a from apogee to perigee, and the satellite at radius r and true anomaly theta.

Sources: the circular-orbit relations, vis-viva equation, and specific energy follow standard astrodynamics texts, principally Curtis [16] and Vallado [17].

Every subsequent chapter depends on a small set of orbital quantities, derived here.

Circular speed. For a satellite of mass m on a circular orbit of radius a, gravity supplies the centripetal force:

μma2=mv2a ⇒ v=μa.

Period and mean motion. From v=2πa/T,

T=2πa3μ,  n=2πT=μa3.

Vis-viva and specific energy. For a general orbit, energy conservation yields the vis-viva relation v2=μ2r−1a, and the total specific energy is

E=v22−μr=−μ2a.

Equation (4.3) is the key to the orbital-decay derivation in Chapter 7: drag removes energy, and a loss of energy means a more negative E, hence a smaller a.

The two-body problem. The circular relations above are special cases of the two-body problem. Newton’s law of gravitation and the second law give the relative acceleration of the satellite with respect to Earth,

r=−μr3 r,  μ=G(M⊕+m)≈GM⊕,

where the approximation holds because the satellite mass is negligible against Earth’s. Taking the cross product of (4.4) with r shows that the specific angular momentum is conserved,

h=r×r,  dhdt=0,

so the motion stays in a plane and the radius vector sweeps equal areas in equal times, which is Kepler’s second law.

The orbit equation. Integrating (4.4) once, using the conserved eccentricity (Laplace-Runge-Lenz) vector, yields the trajectory as a conic section written in focal polar form,

r(θ)=h2/μ1+ecosθ=p1+ecosθ,  p=a1−e2,

where e is the eccentricity, θ the true anomaly measured from perigee, and p the semi-latus rectum. Bound orbits have 0≤e<1; the circular orbit used throughout this study is the special case e=0, for which r=a is constant and (4.1) to (4.3) follow directly.

Kepler’s third law. For an ellipse of semi-axes a and b=a1−e2, the constant areal rate A=h/2 integrated over the enclosed area πab gives the period

T=2πa3μ,

independent of eccentricity, which is why the circular result (4.2) applies to the slightly non-circular reference orbit without correction.

Ground track and revisit. Coverage planning needs the ground track, the locus of sub-satellite points. While the satellite completes one orbit in time T, Earth turns beneath it at ω⊕=7.292×10−5 rad/s, so successive equator crossings step westward by Δλ=ω⊕T. A ground track repeats when an integer number of orbits k fills an integer number d of nodal days, k T=d (2π/ω⊕); for the reference orbit Δλ≈24.5∘ per revolution, and the near-15-orbit day sets the natural revisit cadence over a given ground station (Chapter 14).

Worked example. At h=650 km, a=RE+h=7.021×106 m:

v=3.986×10147.021×106=7534.8 m/s,  T=2π(7.021×106)33.986×1014=5854.8 s=97.6 min.

The orbital speed is about 7.5 km/s and the period about 97.6 minutes, giving roughly 14.8 orbits per day. These values are reproduced exactly by orbital_dc.orbit.velocity(650) and period(650).

## 5. Sun-Synchronous Orbits and J2 Precession

Abstract. This chapter derives the secular nodal precession caused by Earth’s oblateness and the inclination that makes an orbit sun-synchronous, and confirms the reference value of about ninety-eight degrees.

The dawn-dusk sun-synchronous orbit, with the orbital plane held on the day-night terminator so the satellite stays sunlit.

Sources: the J2 secular precession and the sun-synchronous condition follow Vallado [17] and Curtis [16].

A sun-synchronous orbit maintains a near-constant orientation relative to the Sun by exploiting Earth’s oblateness. The dominant non-spherical term in Earth’s gravity, characterized by J2, causes the orbital plane’s right ascension of the ascending node, Ω, to precess. The secular precession rate for a near-circular orbit is

Ω=−32 n J2REa2cosi.

Origin of the precession. Earth’s gravitational potential, expanded in zonal harmonics and kept to the leading oblateness term, is

U(r,ϕ)=−μr 1−J2REr2P2(sinϕ) ,  P2(sinϕ)=123sin2ϕ−1,

where ϕ is geocentric latitude and P2 is the second Legendre polynomial. The term beyond the point-mass field is the disturbing function R. Writing the latitude through the orbital geometry, sinϕ=sini sin(ω+θ) with θ the true anomaly, and averaging R over one revolution removes the short-period terms and leaves the secular part

R=n2J2RE22 1(1−e2)3/21−32sin2i.

Lagrange’s planetary equation for the node connects this averaged potential to the drift of Ω,

Ω=1na21−e2 sini ∂R∂i.

Carrying out the derivative of (5.3) with respect to i gives ∂R/∂i∝sinicosi, and the sini in the denominator of (5.4) cancels, leaving the cosi dependence of (5.1). The result is exact at first order in J2 for any eccentricity if a is replaced by the semi-latus rectum p=a(1−e2); for the near-circular reference orbit p≈a and (5.1) follows directly. The sign is negative for prograde orbits (i<90∘), so the node regresses, and positive for retrograde orbits, which is what makes synchronization with the Sun possible.

A sun-synchronous orbit chooses the inclination i so that this precession equals Earth’s mean rate about the Sun, Ωsun=2π/(365.2422 d)=1.991×10−7 rad/s. Setting Ω=Ωsun and solving,

cosi=−2 Ωsun a23 n J2 RE2.

The negative sign forces i>90∘, a retrograde, near-polar orbit. At 650 km, n=1.072×10−3 rad/s and (5.5) gives cosi=−0.139, hence i=98.0∘, matching the reference mission and the value returned by orbital_dc.orbit.sso_inclination(650). By construction the corresponding nodal precession is 0.9856 degrees per day, exactly one turn per tropical year, which is the defining property of the orbit. The required inclination rises slowly with altitude, from about 96.3∘ at 200 kilometers to 98.6∘ at 800 kilometers, because the a2 factor in (5.5) outpaces the fall in n. The dawn-dusk variant orients the orbital plane near Earth’s terminator, which (Chapter 6) yields nearly continuous sunlight and minimal thermal cycling, a decisive advantage for a high-power, thermally sensitive payload.

Recovering the mean motion. The precession rate (5.1) and the inclination condition (5.5) both require the mean motion n, which follows from Kepler’s third law for a two-body orbit. Equating the gravitational force to the centripetal requirement for a circular orbit of radius a, μm/a2=m n2a, gives the relation between angular rate and size,

n=μa3,

where μ=GME is Earth’s gravitational parameter and a=RE+h for a circular orbit at altitude h. The orbital period is then T=2π/n=2πa3/μ. Because n scales as a−3/2, doubling the altitude lengthens the period and slows the satellite, which is the falling factor that competes with the rising a2 term in (5.5).

The worked inclination at the reference altitude. Take the reference orbit at h=650 km, so a=RE+h≈6378+650=7028 km. Substituting into (5.6) with μ=3.986×1014 m3/s2 returns n=1.072×10−3 rad/s, the value already quoted in the chapter. Feeding this n, together with J2=1.083×10−3 and RE=6378 km, into the synchronization condition (5.5) yields cosi=−0.139, hence i=98.0∘, in agreement with the reference mission. The two relations therefore form a closed chain: altitude fixes n through (5.6), and n fixes the inclination through (5.5).

Sensitivity to altitude. Differentiating (5.5) shows how tightly the inclination must be held as the altitude is chosen. Writing cosi∝a2/n=a2a3/2=a7/2 after inserting (5.6), the synchronization condition becomes

cosi=−2 Ωsun3 J2 RE2 a7/2μ,

which makes explicit the strong a7/2 growth that drives i from 96.3∘ toward 98.6∘ across the low-Earth-orbit band. The design implication is that the sun-synchronous inclination is a slowly varying but firmly dictated function of the chosen altitude; once a thermal or power requirement sets the altitude, the inclination is not a free parameter but is fixed to within a small fraction of a degree by (5.7).

## 6. Eclipse Geometry and the Dawn-Dusk Advantage

Abstract. This chapter derives the beta angle and the sunlit fraction of the orbit from the cylindrical-shadow geometry, and shows why the dawn-dusk orbit stays sunlit year round.

The beta angle between the orbit plane and the Sun line, set against Earth’s shadow; a high beta keeps the orbit out of eclipse.

Sources: the beta-angle and eclipse-fraction geometry follow Curtis [16] and the SMAD mission-design text [18].

The fraction of each orbit spent in Earth’s shadow is set by the beta angle β, the angle between the orbital plane and the Sun line. A satellite avoids eclipse entirely when |β| exceeds the critical value at which the orbit just grazes Earth’s limb:

β*=arcsin​RERE+h.

For |β|<β*, the sunlit fraction of the orbit is

fsun=1−1πcos−1​h2+2REh(RE+h)cosβ.

Derivation from the cylindrical shadow. Treat Earth’s shadow as a half-infinite cylinder of radius RE pointing away from the Sun, a good approximation in low Earth orbit where the penumbra is narrow and the Sun is effectively at infinity. A point on the circular orbit of radius r=RE+h lies in shadow when its distance from the shadow axis is less than RE and it is on the anti-sun side. Resolve the orbit into the shadow frame: the component of the orbit radius perpendicular to the Sun line, for an orbit whose plane makes angle β with that line, traces an ellipse of semi-axes r and rcosβ. The satellite is eclipsed over the arc where this projected position falls within the cylinder, and the half-angle ψ of that arc, measured from the anti-solar point, satisfies

cosψ=r2−RE2r cosβ=h2+2REh(RE+h)cosβ.

The eclipse occupies the fraction fE=ψ/π of the orbit (the full eclipsed arc is 2ψ out of 2π), and the sunlit fraction is fsun=1−fE, which is (6.2). The expression is real only while the bracket does not exceed one; the threshold where cosψ=1 recovers the critical beta of (6.1), at which the eclipse arc shrinks to zero.

Annual variation of the beta angle. The beta angle is not fixed: it depends on the angle between the orbit plane and the instantaneous Sun direction. For an orbit of inclination i and node Ω, with solar ecliptic longitude λ⊙ and obliquity ε=23.44∘,

sinβ=cosλ⊙sinΩsini−sinλ⊙cosεcosΩsini+sinλ⊙sinεcosi.

A sun-synchronous orbit holds Ω at a fixed local time by construction, so the seasonal swing of λ⊙ is what moves β. For the dawn-dusk case, where the node is set near the six o’clock line so the orbit plane contains the terminator, (6.4) reduces to |β| oscillating between about 66.6∘ and 90∘ over the year, the lower bound being 90∘−ε.

At 650 km, β*=arcsin(6371/7021)=65.1∘. Because the dawn-dusk orbit keeps |β| above 66.6∘ at all times, it stays above this 65.1∘ threshold year round, so the satellite never enters a deep eclipse and fsun→1: illumination exceeds ninety-five percent, the battery need is minimal (Chapter 11), and the thermal environment is nearly steady, justifying the quasi-steady thermal treatment of Chapter 10. This is the single most favorable environmental choice available to the architecture, and it is robust to the seasonal beta swing rather than dependent on a single favorable date.

Fig. 6.1 Critical beta angle versus altitude, and eclipse fraction versus beta at 650 km. A dawn-dusk orbit holds the beta angle high, so the satellite stays sunlit.

Detailed reduction of the projected geometry. It is worth retracing how the shadow-cylinder condition of (6.3) emerges from vectors, because the same projection underlies the power and thermal budgets of later chapters. Place the origin at Earth’s center and let s be the unit vector toward the Sun. A point on the circular orbit is r(u)=r e(u), where u is the argument measured from the ascending point of the orbit and e sweeps the orbit plane. The component of r along the Sun line is r∥=r⋅s, and the perpendicular distance from the shadow axis is

d⟂(u)=r2−(r⋅s)2.

By the definition of the beta angle, the orbit plane is tilted so that the projection of r onto the Sun line has amplitude rcosβ over a revolution; writing the in-plane phase as θ measured from the sub-solar direction gives r⋅s=rcosβ cosθ, so that

d⟂(θ)=r1−cos2β cos2θ.

Solving for the eclipse arc. Eclipse requires both d⟂<RE and the anti-solar half cosθ<0. Setting d⟂=RE at the boundary and solving (6.6) for the phase θ=π−ψ at which the orbit crosses the cylinder wall,

cos2β cos2ψ=1−RE2r2=r2−RE2r2,

and taking the positive root recovers cosψ=r2−RE2 /(rcosβ), which is exactly (6.3). The eclipse exists only when the right side of (6.7) does not exceed cos2β; the equality cosψ=1 reproduces cosβ=r2−RE2/r=1−(RE/r)2, equivalent to sinβ*=RE/r of (6.1). The two relations are therefore one statement viewed at its limit.

Eclipse duration in time. For mission power sizing the arc must be converted to a duration. The orbital period follows from Kepler’s third law,

T=2πr3μ⊕,

with μ⊕ the Earth gravitational parameter, and the eclipse interval is the eclipsed fraction times the period,

tE=2ψ2π T=ψπ T.

Worked example. At h=650 km, r=7021 km and β*=65.1∘, as stated in the chapter. For the dawn-dusk orbit the beta angle never falls below 66.6∘, so |β|>β* throughout the year; (6.7) then has no real solution for ψ, the eclipse arc vanishes, tE=0, and the sunlit fraction fsun→1, consistent with the chapter’s statement that illumination exceeds ninety-five percent.

Design implication. Because the eclipse duration collapses to zero whenever |β| exceeds β*, holding the dawn-dusk node removes the eclipse load term from the power and energy-storage budgets, so the battery is sized for contingency and load transients rather than for nightly discharge.

## 7. The Neutral Atmosphere and Orbital Decay

Abstract. This chapter derives the orbital-decay rate from the energy balance under atmospheric drag and integrates it to a mission lifetime, a result that decides compliance with the five-year disposal rule.

Atmospheric drag slowly removes orbital energy, so the orbit spirals inward; the natural lifetime is about two decades at moderate solar activity.

Sources: density from the NRLMSIS empirical model [11]; the drag and orbit-averaged decay treatment follows Curtis [16] and the space-environment texts of Tribble [22] and Pisacane [23].

Even at 650 km a tenuous atmosphere remains, and the drag it exerts slowly lowers the orbit. This chapter derives the decay rate and integrates it to a mission lifetime, a result that proves decisive for regulatory compliance.

Drag force and specific deceleration. A body of cross-section A moving at speed v through gas of density ρ feels a drag force opposing its velocity:

FD=12 ρ v2 CD A,

with CD≈2.2 in the free-molecular regime. The specific (per-mass) deceleration is aD=FD/m=12ρv2CD(A/m).

Derivation of the decay rate. Drag does negative work at the rate (force times speed, per unit mass):

dEdt=−aD v=−12 ρ CD Am v3.

For a near-circular orbit E=−μ/2a, so dE/dt=(μ/2a2) a. Equating with (7.2) and substituting v=μ/a so that v3=(μ/a)3/2:

μ2a2dadt=−12ρCDAmμa3/2 ⇒ dadt=− CD Am ρ(h) μ a.

Because ρ(h) rises steeply as the orbit lowers, the decay accelerates and terminates in a rapid re-entry. The per-revolution contraction is Δarev=a T=−2πCD(A/m)ρa2.

Atmosphere model and solar-cycle uncertainty. The density ρ(h) is taken from the NRLMSIS 2.1 empirical model, interpolated log-linearly between anchors at 600, 650, 700, and 800 km for three levels of solar activity. The crucial feature is that density at this altitude varies by roughly a factor of one hundred between solar minimum and solar maximum. This single fact dominates the uncertainty in lifetime, drag make-up, and station-keeping, and is therefore propagated explicitly as a band rather than collapsed to a point.

Numerical solution and result. Equation (7.3) is integrated by adaptive forward Euler to a re-entry altitude of 120 km. For the reference bus, with an area-to-mass ratio of 0.0084 m2/kg and CD=2.2, the natural lifetime at 650 km is approximately twenty-two years at moderate solar activity, about four years at solar maximum, and over three hundred years at solar minimum (Fig. 7.1). The regulatory consequence is severe: the United States Federal Communications Commission now requires disposal within five years (Chapter 15), so only a satellite that happens to launch into a solar maximum approaches passive compliance. Active de-orbit propulsion is therefore mandatory, not optional, a conclusion that propagates into the mass and economic budgets.

Fig. 7.1, Natural orbital lifetime versus altitude with the solar-cycle uncertainty band; at 650 km the realistic lifetime fails the five-year disposal rule.

Scale height and the local density gradient. The steep altitude dependence invoked above follows from hydrostatic balance in an isothermal layer. Setting the upward pressure gradient against gravity, dp/dh=−ρg, and using the ideal-gas relation p=ρkBT/m with mean molecular mass m, one eliminates the pressure to obtain a first-order equation for the density:

1ρdρdh=−m gkBT≡−1H,  H=kBTm g.

Integrating (7.4) over an interval where H is treated as constant gives the local exponential profile

ρ(h)=ρ(h0) exp​−h−h0H.

The quantity H is the scale height, the vertical distance over which density falls by a factor e. This same exponential structure justifies the log-linear interpolation between the tabulated NRLMSIS anchors, because a straight line on a logarithmic density axis is precisely the form of (7.5).

Closed-form lifetime for a locally exponential atmosphere. A compact estimate of lifetime follows by inserting (7.5) into the decay relation (7.3). Writing B=CD(A/m) for the ballistic factor and separating variables,

daa exp[−(a−a0)/H]=−B ρ0 μ dt.

Because the exponential dominates the slowly varying a, the integral is concentrated near the starting radius, and the time to descend by one scale height reduces to

tlife≈HB ρ0 μ a0.

Equation (7.7) shows transparently that lifetime scales inversely with both the ballistic factor and the ambient density, the latter being the term that swings by two orders of magnitude across the solar cycle.

Worked example. Take the reference bus with B=CD(A/m)=2.2×0.0084≈0.0185 m2/kg at a0≈7028 km and μ=3.986×1014 m3/s2, so that μa0≈5.3×1010 m2/s. With a representative scale height of order tens of kilometres, (7.7) returns lifetimes in the decade-to-century range as the density ρ0 is swept across its solar-cycle band, consistent with the integrated result of roughly twenty-two years at moderate activity reported above.

Design implication. Equation (7.7) makes the sensitivity explicit; since lifetime falls linearly with the area-to-mass ratio and with ambient density, a designer cannot passively meet the five-year rule across the solar cycle, and the de-orbit propellant budget must be sized to the worst case rather than to a nominal density.

## 8. The Ionizing-Radiation Environment

Abstract. This chapter treats the two distinct radiation effects, accumulated dose behind shielding and stochastic single-event upsets, and shows that dose is comfortably survivable while upsets impose a bounded throughput tax.

Radiation shielding and single-event upsets: aluminium attenuates the accumulated dose, while error correction and checkpointing handle upsets.

Sources: trapped-particle fluxes from the AE8/AP8 [25] and AE9/AP9 [26] models; dose-depth via SHIELDOSE [27]; environment and effects from Tribble [22], Pisacane [23], and Hastings and Garrett [24].

Two physically distinct radiation effects matter: the slow accumulation of total ionizing dose, which is a shielding problem, and the stochastic flipping of bits by energetic particles, which is a reliability and throughput problem.

Dose versus shielding. Behind aluminium of thickness x, the trapped-electron contribution to dose attenuates quasi-exponentially atop a floor set by penetrating protons and bremsstrahlung:

D(x)=D0 e−x/λ+D∞,  λ≈1.7 mm.

The constants are calibrated so that the deep-shield floor matches the proton-dominated value of roughly 100 to 150 rad(Si) per year measured by flight dosimeters in comparable sun-synchronous orbits and consistent with the reference mission’s own estimate. The canonical engineering tool for this curve is SHIELDOSE-2 with an AE8/AP8 or AE9/AP9 trapped-particle environment.

Result and margin. At 10 mm of aluminium, the five-year cumulative dose is approximately 1.06 krad(Si). The reference mission reports that its processor’s most sensitive subsystem, the high-bandwidth memory, first showed irregularities at 2 krad(Si), about 2.7 times the 750 rad(Si) five-year requirement, and exhibited no hard failures up to the maximum tested dose of 15 krad(Si). The margin is therefore large: roughly a factor of two to the first-irregularity threshold and an order of magnitude to the no-hard-failure level. Total ionizing dose is comfortably survivable for a five-year mission with sensible shielding.

Fig. 8.1 Cumulative five-year ionizing dose versus aluminium shielding, with the memory tolerance and the mission requirement marked. Ten millimeters of aluminium keeps the dose well below the first-irregularity threshold.

Single-event effects. Energetic protons and heavy ions, concentrated in the South Atlantic Anomaly and the polar horns, deposit charge that flips memory and logic states. The upset rate scales as RSEU=σSEU ϕ Nbits, the product of the per-bit cross-section, the particle flux, and the number of bits. The key enabling insight, treated quantitatively in Chapter 17, is that AI inference tolerates bit errors far better than traditional high-performance computing; the optimal architecture is therefore error-correcting memory with checkpointing, not full hardware redundancy. The residual cost is a throughput tax of order ten to twenty percent, which must be carried explicitly in any efficiency figure.

Integrating the dose-depth law. The cumulative dose quoted for a fixed shielding thickness follows from integrating the dose rate of Eq. (8.1) over the mission lifetime T. Because the constants D0, λ, and D∞ are taken as time-independent for a stable orbit, the integral is elementary:

D(x,T)=0TD(x) dt=D0 e−x/λ+D∞T.

The two terms separate cleanly into an electron component that the shield attenuates and a floor that it does not. The shielding design problem is to drive the first term below the second so that additional aluminium yields diminishing returns. Setting the attenuated electron term equal to a chosen fraction ϵ of the floor gives the thickness at which further mass is no longer productive:

x*=λln​D0ϵ D∞.

Worked dose estimate. The chapter states a five-year dose of approximately 1.06 krad(Si) at x=10 mm. Treating the deep-shield rate as dominated by the floor, the floor itself is recovered directly from Eq. (8.2):

D∞≈D(x,T)T=1.06 krad(Si)5 yr≈212 rad(Si) yr−1,

which sits in the same range as the 100 to 150 rad(Si) per year cited for comparable sun-synchronous orbits once the small residual electron term at 10 mm is included. The shielding margin is governed not by the attenuation length, which is fixed by the spectrum, but by the floor, so beyond roughly one to two centimetres of aluminium the design trades shield mass for negligible dose reduction.

Upset rate and the throughput tax. Expanding the upset relation RSEU=σSEU ϕ Nbits, the throughput tax τ is the share of processor cycles reclaimed by error correction and checkpoint replay, namely the upset rate times the cycles charged per upset, divided by the cycle rate rcyc:

τ=cECC RSEUrcyc=cECC σSEU ϕ Nbitsrcyc,

where cECC is the average cycles charged per detected upset and rcyc is the processor cycle rate, so τ is dimensionless. The tax scales linearly with flux, so it peaks in the South Atlantic Anomaly and the polar horns and falls to a small floor elsewhere, consistent with the bounded ten to twenty percent figure carried in the efficiency budget.

Design implication. Shield to the dose floor rather than to an arbitrary thickness, since mass past a centimetre or two buys almost no margin; then size the compute overhead from Eq. (8.5) so that the upset tax is budgeted explicitly rather than absorbed silently into throughput claims.

## 9. The Orbital-Debris Environment

Abstract. This chapter quantifies the debris hazard at the reference altitude: the catastrophic-impact probability for the cluster and the way a single fragmentation couples to neighbours as the spacing tightens.

The formation paradox: one fragmentation sprays debris into the volume the neighbours occupy, with the neighbour-hit probability scaling as one over spacing squared.

Sources: flux from NASA ORDEM 3.2 [8] and ESA MASTER [9]; the collision-frequency and cascade framework follows Kessler and Cour-Palais [28] and Klinkrad [29].

Low Earth orbit is increasingly congested, and the 650 km sun-synchronous shell is among the most crowded. The European Space Agency’s environment models and NASA’s ORDEM 3.2 are the authoritative flux tools; the latter projects the environment forward to 2050 and covers the relevant altitude and epoch. Catastrophic risk is driven by objects larger than one centimeter, which are lethal at orbital closing speeds yet too small to track and avoid individually.

Impact probability as a Poisson process. Impacts are rare, independent events. Dividing a mission into n short intervals, each with hit probability p=ΦA Δt (flux times cross-section times time), the number of impacts is binomial; in the limit n→∞ with np→Λ it becomes Poisson with mean Λ=ΦANt, where N is the number of satellites. The probability of at least one catastrophic impact in the cluster is

P(K≥1)=1−e−ΦANt.

With a model-class flux for objects larger than one centimeter of Φ=3×10−5 m−2yr−1, a tumbling-averaged silhouette of A=15 m2 per satellite, N=81, and t=5 years, the mean is Λ=0.182 and the probability is about seventeen percent, with a model band from six to forty-six percent. The development of the cascade mechanism, the reason the tight formation is dangerous, is given in Chapter 13 and the risk synthesis. The flux used here is model-class; a flight program would run the official ORDEM/MASTER tools for the exact orbit and epoch, as noted in Chapter 24.

From the binomial limit to the Poisson mean. The passage from independent short intervals to a single closed-form probability deserves the intermediate steps. Partition the mission into n equal slices of duration Δt=t/n. If p=ΦA Δt is the per-slice, per-satellite hit probability, the count of catastrophic impacts across N satellites and n slices follows a binomial law with trial number m=Nn and success probability p. The probability of zero impacts is

P(K=0)=(1−p)m=1−ΦANtmm.

Taking the limit m→∞ at fixed product mp=ΦANt, and using limm→∞(1−x/m)m=e−x, the zero-impact probability collapses to e−ΦANt. The complement is the at-least-one expression already stated as Equation (9.1); the grouped mean is

Λ=mp=ΦANt,

which makes explicit that the hazard scales linearly in flux, silhouette, fleet size, and mission length. This linearity is the reason a long-lived, many-node cluster cannot escape the environment by clever scheduling; only the four factors in Equation (9.3) move the risk.

Worked mean and the small-Λ reading. Substituting the chapter’s reference values, Φ=3×10−5 m−2yr−1, A=15 m2, N=81, and t=5 yr, gives

Λ=(3×10−5)(15)(81)(5)=0.182,

in agreement with the value quoted in the text. Because Λ is below unity, the series expansion 1−e−Λ=Λ−Λ2/2+⋯ shows that P(K≥1) sits slightly under Λ itself, here near seventeen percent, while the expected number of distinct impacts is just Λ. The two coincide only to first order, so reporting the mean and the probability separately is appropriate.

Per-satellite versus fleet risk. It is often useful to separate the single-vehicle exposure from the aggregate. Writing λ1=ΦAt for one satellite, the fleet survival probability is the product of independent survivals,

P(K=0)=e−λ1N=e−Nλ1,

which recovers Equation (9.1) and confirms the independence assumption is what permits the simple exponent Nλ1. The design implication is direct: since fleet risk grows as the product Nλ1, halving the tumbling-averaged silhouette A buys the same risk reduction as halving either the node count or the mission duration.

# PART III, SPACECRAFT SUBSYSTEMS

## 10. Thermal Control

Abstract. This chapter develops the thermal model, the linear scaling of radiator area with power and the sharper wall at the chip interface, and identifies the power level above which a passive path can no longer cope.

The passive conduction path from the junction to the radiator, dominated by the junction-to-case resistance.

Sources: radiative and conductive relations from Incropera and DeWitt [21]; spacecraft-specific practice from the Gilmore thermal-control handbook [20] and ECSS-E-ST-31C [34].

Thermal control is where the orbital data center is most often assumed to be hard and is in fact solved at the single-node scale, but it does not scale for free, and a second, sharper limit appears at the chip interface.

From Planck’s law to Stefan-Boltzmann. A blackbody emits spectral radiance

B(λ,T)=2hPc2λ5 1exp(hPc/λkBT)−1.

Integrating over the emitting hemisphere (the factor π from ∫cosθ dΩ) and over all wavelengths gives the hemispherical emissive power,

Eb(T)=π​0∞B(λ,T) dλ=σT4,  σ=2π5kB415hP3c2.

A gray surface of emissivity ε radiating to the cosmic-microwave-background sink at TCMB rejects Qrad=εσA(T4−TCMB4); since TCMB4/T4∼10−8 at room temperature, the sink term is negligible.

Radiator sizing. A deployed radiator radiates from both faces and loses a fraction fp of its output to absorbed solar, albedo, and Earth-infrared loads (the external loads of Chapter 8 and the view factor below). The net rejected flux per unit one-side area is εσT4ns(1−fp) with ns=2, so the required area for a heat load Q at operating temperature T is

Arad=Qε σ T4 ns (1−fp).

This is linear in Q: there is no economy of scale in radiative cooling, a fact with profound architectural consequences. Worked example: at 1 MW, 20 C, ε=0.90, ns=2, fp=0.12, the per-side flux is εσT4=376.9 W/m2, the net flux is 663.3 W/m2, and Arad=1508 m2. This is of the same order as the industry rule of roughly 1,200 m² per megawatt and explains why Starcloud’s gigawatt vision implies multi-kilometer panels.

Fig. 10.1 Radiator equilibrium temperature versus heat flux, and the radiator area required for one megawatt versus operating temperature. Running hotter reduces area at the cost of silicon lifetime.

External loads and view factor. A surface at orbital radius r sees Earth through the view factor F=12[1−1−(RE/r)2], equal to 0.290 at 650 km. The absorbed loads are Qalbedo=S0 aalb F αs A and QIR=qIR F ε A; for an edge-on radiator the direct-solar term vanishes. With αs=0.15 and A=4 m2 these are about 71 W and 247 W respectively, which the design lumps to roughly 100 W of parasitic load.

Fig. 10.2 Earth view factor versus altitude, and the combined albedo, Earth-infrared, and solar load on a four-square-meter surface. External loads set the parasitic fraction in the radiator sizing.

Inverse problem and Newton-Raphson. When the area is fixed and the temperature is sought, (10.3) becomes a quartic, solved by Newton-Raphson on f(T)=Q−εσAT4 with f′(T)=−4εσAT3, converging in two to four iterations from T0=300 K. The first report’s baseline (1450 W on 4 m², ε=0.85, single-sided) yields a radiator temperature of 21.4 C.

The conductive chain and the interface wall. Heat first travels from the junction to the radiator through series resistances, Tj=Trad+Q Rth. With the passive chain Rth=0.30 K/W, the rise across the interface alone is P×0.30: 90 C at 300 W, 210 C at 700 W, and 360 C at 1200 W. Because the entire junction budget is 125 C, any chip with P Rth>125 C, that is, P>417 W, cannot be cooled passively at any radiator size. Above this threshold, chip-level liquid or loop-heat-pipe cooling is mandatory. This is why flown systems (Starcloud, Axiom) use pumped loops rather than the passive vapor chamber the first report assumed.

Fig. 10.1, (a) Radiator area is linear in heat load; (b) the passive thermal chain saturates above ~400 W, forcing chip-level active cooling.

Deriving the radiator scaling from the energy balance. The sizing relation (10.3) follows from a steady-state energy balance on the panel. In equilibrium the dissipated electrical load Q equals the net radiated power, while the absorbed external loads of Chapter 8 reduce the useful output by the fraction fp. Writing the balance for a two-sided panel of one-side area Arad,

Q=ε σ ns Arad (T4−TCMB4) (1−fp).

Dropping the sink term, since TCMB4/T4∼10−8, and solving for the area returns (10.3) exactly. The linearity in Q is therefore structural, not incidental; the area carries no exponent that would let a larger plant reject heat more cheaply per watt.

Sensitivity to operating temperature. Because the rejected flux varies as T4, the marginal benefit of running the silicon hotter is large. Differentiating (10.3) at fixed Q, ε, ns, and fp,

dAradArad=−4 dTT.

A one percent rise in absolute operating temperature reduces the required area by about four percent. This is the quantitative form of the trade noted in Fig. 10.1: area is bought back by raising T, at the cost of junction lifetime.

The conductive chain and the passive ceiling. The series resistance model Tj=Trad+Q Rth sets the second limit. Requiring the junction to stay within its allowable rise ΔTmax=Tj,max−Trad bounds the power a passive chain can carry,

Pmax=ΔTmaxRth.

Worked example. Taking the chapter’s junction budget ΔTmax=125 C and the passive chain Rth=0.30 K/W,

Pmax=125 C0.30 K/W≈417 W.

This reproduces the 417 W threshold and confirms that no radiator size relaxes it, since Rth acts before the heat reaches the panel.

Design implication. Radiator mass scales linearly with plant power and the chip interface caps any single device near 417 W passively; an orbital data center must therefore plan for kilometer-scale panels and adopt pumped-loop or loop-heat-pipe cooling at the chip from the outset.

## 11. Electrical Power

Abstract. This chapter sizes the electrical-power subsystem for the dawn-dusk orbit, where near-continuous sunlight makes the battery small and the array sizing the dominant term.

Power flow on the dawn-dusk orbit: the array feeds the bus and loads, and because the orbit is sunlit for more than ninety-five percent of each revolution the battery is barely exercised.

Sources: solar-array and power-budget sizing from SMAD [18] and the Gilmore handbook [20]; cell and battery parameters from vendor datasheets [12].

The power subsystem converts sunlight to usable electrical power and buffers the brief eclipse.

Array power. The electrical power delivered to the load is the incident solar power times the cell efficiency (which falls with temperature and degrades with radiation), the panel packing factor, and the array-to-load path efficiency:

P=S0 A η(T,life) κpack ηpath,  η(T,life)=ηlife 1+kT(T−28).

Triple-junction GaInP/GaAs/Ge cells deliver about 29.6 percent at beginning of life and 27.5 percent after a five-year fluence, with a temperature coefficient kT≈−0.0023 per C, packing κpack=0.85, and path efficiency ηpath=0.88. Worked example: at end of life and 70 C, η=0.275[1−0.0023(70−28)]=0.248, so P=1361×1×0.248×0.85×0.88=253 W/m2 at the load.

Battery and the dawn-dusk lever. To carry the load PL through an eclipse of duration te at depth-of-discharge DoD and specific energy esp, the battery mass is mbat=PLte/(DoD esp). In a dawn-dusk orbit, te is small and seasonal, so the battery is light, a major mass saving over a generic low orbit with sixteen deep cycles per day. This is the power-subsystem expression of the dawn-dusk advantage established in Chapter 6.

Fig. 11.1, Array area versus load, and triple-junction degradation over the mission.

Deriving the temperature-and-life efficiency factor. The cell efficiency in equation (11.1) combines two independent degradation paths, and it is worth separating them. The radiation-driven fall from beginning-of-life efficiency ηBOL to the end-of-life value ηlife is multiplicative over the mission, so for an annual degradation rate d acting over N years the surviving fraction is

ηlife=ηBOL (1−d)N.

Linearizing the temperature term about the reference temperature of 28 C, the relative efficiency change is the temperature coefficient times the excursion, which recovers the bracketed factor of (11.1):

η(T)−ηlifeηlife=kT (T−28).

Using ηBOL=0.296 and ηlife=0.275 over five years, equation (11.2) gives an implied annual rate $d = 1 - (0.275/0.296)^{1/5} pprox 0.0146$, that is about 1.5 percent per year, consistent with the five-year fluence quoted for the triple-junction cell.

Array equilibrium temperature. Because η depends on T, the operating temperature is itself set by a radiative balance and should be solved, not assumed. Equating absorbed solar flux to emitted thermal flux from both faces of the panel, with absorptance α, front and back emittances εf+εb, and the electrical power drawn off the front face, gives

α S0=(εf+εb) σ T4+S0 η κpack,

so that, solving for the equilibrium temperature,

T=S0 (α−η κpack)(εf+εb) σ1/4.

Equations (11.5) and (11.3) form a small fixed point: a hotter panel lowers η, which in turn raises the equilibrium temperature slightly. One or two iterations from the 70 C operating point used in the worked example of (11.1) are sufficient for sizing.

Array sizing closure. Inverting the load relation of (11.1) gives the collecting area required to meet a load PL,

A=PLS0 η(T,life) κpack ηpath,

which at the end-of-life value of 253 W/m2 delivered to the load means each megawatt of demand needs roughly 3953 m2 of array.

Design implication. Because the array area scales inversely with the temperature-depressed end-of-life efficiency, the panel thermal design and the radiation-tolerant cell selection are leveraged directly into deployed area and structural mass; cooling or de-rating the array is a sizing decision, not a comfort margin.

## 12. Structures and Mass

Abstract. This chapter estimates the structural and mass properties, the inertia tensor, the structural mass set by launch loads, and the panel modal frequency, that feed the attitude and integration analyses.

Sources: mass-growth allowances from AIAA S-120 [14]; structural and modal relations from SMAD [18] and Sidi [33].

The structure carries launch loads, sets the spacecraft inertia (which feeds attitude control), and anchors the mass budget that closes the whole design.

Inertia from geometry. A uniform box of mass m and dimensions dx,dy,dz has principal moments Ix=m12(dy2+dz2), and cyclic permutations. A deployed panel of mass m, length L, at offset o from the bus axis contributes m(L2/12+o2) by the parallel-axis theorem. Summing the bus, two solar wings, and the radiators gives a diagonal inertia tensor; for the reference satellite the maximum principal moment is on the order of a few hundred kilogram-meters squared, and the maximum minus minimum difference feeds the gravity-gradient torque of Chapter 13.

Structural mass and launch loads. Primary structure is sized as a fraction (typically fifteen to twenty-five percent) of the supported mass, and checked against the quasi-static launch load: the stress in a primary member is σstress=m gload g0/Amember, and the margin is the yield strength divided by this stress, required to exceed roughly 1.25 with margin. For aluminium at an 8.5-g quasi-static load, modest member areas keep the margin comfortable.

Panel fundamental frequency. Deployed panels must be stiff enough that their fundamental mode does not couple to the attitude-control bandwidth. For a plate of side a, thickness t, and areal mass ρA,

f1=β2πDρA a4,  D=Et312(1−ν2),

with β a boundary-condition constant. Thin, large deployed panels are inherently low-frequency, which is why the optical-pointing solution (Chapter 13) relies on a fine-steering mirror rather than on slewing the whole flexible structure.

Mass closure. The integrated model (Chapter 18) sums structure, thermal, power, attitude, shielding, avionics, payload, harness, and propellant into a dry and a launch mass, and from the dry mass and a drag area computes the ballistic coefficient that feeds the decay and Δv analyses. The reference four-chip satellite closes at roughly 270 to 290 kg dry.

Fig. 12.1 Reference satellite dry-mass breakdown by subsystem, closed self-consistently by the integrated model.

Derivation of the box inertia. The principal moment of a uniform rectangular solid follows from integrating the mass element over the body. For rotation about the x axis the relevant distance from the axis is r2=y2+z2, so with uniform density ρ=m/(dxdydz),

Ix=V​(y2+z2) ρ dV=ρ−dx/2dx/2​​−dy/2dy/2​​−dz/2dz/2(y2+z2) dz dy dx.

Each quadratic integral over a span d centred on the origin yields −d/2d/2y2 dy=d3/12. Carrying the three integrations and grouping the surviving factors gives

Ix=ρ dxdy312dz+dydz312=m12dy2+dz2,

which is the chapter’s stated result, with the y and z moments following by cyclic permutation.

Parallel-axis contribution of an offset panel. A panel of mass m has a moment Icm=mL2/12 about its own centroidal axis. Translating that axis a distance o to the bus axis adds the point-mass term mo2, so

Ipanel=Icm+mo2=mL212+o2,

confirming the relation used to assemble the diagonal tensor from bus, wings, and radiators.

Worked launch-load check. Take a single primary member of area Amember supporting a tributary mass m at the quasi-static load gload=8.5 stated in the chapter. The member stress is σstress=m gload g0/Amember, and the strength margin against a yield strength σy is

MS=σyFs σstress−1=σy AmemberFs m gload g0−1,

where Fs is the factor of safety. Setting MS=0 inverts to the minimum member area,

Amember, min=Fs m gload g0σy,

so the required cross-section scales linearly with supported mass and load factor and inversely with material strength.

Design implication. Because inertia grows with the square of panel length and offset while the launch-driven member area grows only linearly with mass, large deployed structures are dominated by their inertial and modal penalties rather than by static strength; the mass budget should therefore be tightened at the geometry stage, before member sizing.

## 13. Attitude Determination and Control; Optical Pointing

Abstract. This chapter treats attitude determination and control and the much finer pointing the optical links demand, showing why a fast-steering mirror is required on top of the spacecraft bus.

Coarse body pointing combined with a fast-steering mirror, which supplies the sub-microradian correction the optical link requires.

Sources: attitude dynamics and control from Wertz [32] and Sidi [33]; optical-pointing requirements from Hemmati [30].

The attitude subsystem must hold the bus steady against disturbance torques and, far more demandingly, point laser links to sub-microradian accuracy.

Disturbance torques. The dominant low-orbit torques are gravity-gradient and aerodynamic:

τgg=32 n2 |Iz−Ix| sin2θ,  τaero=12ρv2CDA ℓcp-cg,

where ℓcp-cg is the offset between the center of pressure and the center of mass. With the geometry-derived inertia of Chapter 12, the gravity-gradient torque at 650 km is of order 10−4 N m. Reaction wheels absorb these torques and are desaturated by magnetorquers against Earth’s field.

Optical pointing and the fine-steering mirror. A laser inter-satellite link requires pointing accuracy below one microradian. The diffraction-limited beam divergence is Θ≈2.44 λ/D, and a pointing error θerr relative to the beam divergence θdiv costs received power Lpoint=exp(−2θerr2/θdiv2). A good star tracker provides attitude knowledge of about two arcseconds, or roughly ten microradians, an order of magnitude too coarse. The bus alone cannot hold the beam; a coarse gimbal plus a fine-steering mirror, closing its own loop on the received beacon, is mandatory. The reassuring corollary is that the 150-m neighbor links are geometrically trivial (a twenty-microradian beam is millimeters wide at 150 m); the demanding pointing case is the ground link and any long inter-satellite hop.

Derivation of the gravity-gradient torque. The gravity-gradient torque arises because the gravitational pull on a mass element varies with distance from Earth, so a non-spherical inertia distribution feels a net restoring couple. For a rigid body whose principal axes make small angles with the orbit frame, the torque about the body axes is

τgg=3n2 r×(I r),

where n=μ/a3 is the orbital mean motion, r is the local vertical unit vector, and I is the inertia tensor. Resolving r in the principal frame for a single-axis pitch offset θ gives components r=(sinθ, 0, cosθ). Forming I r=(Ixsinθ, 0, Izcosθ) and taking the cross product leaves a single nonzero component,

τgg=3n2(Iz−Ix)sinθcosθ=32 n2 (Iz−Ix) sin2θ,

using 2sinθcosθ=sin2θ. The absolute value in (13.1) reflects that the magnitude, not the sign, sets the wheel-sizing demand. The torque vanishes when the minor axis is vertical and peaks at θ=45∘, which is why a stable bus is flown with its minimum-inertia axis along the local vertical.

Mean motion at the reference altitude. With μ=3.986×1014 m3 s−2 and a circular orbit at 650 km, the semi-major axis is a=6378+650=7028 km, so

n=3.986×1014(7.028×106)3≈1.07×10−3 rad s−1.

The factor n2≈1.1×10−6 s−2 multiplied by an inertia asymmetry of order 102 kg m2 reproduces the order 10−4 N m gravity-gradient torque quoted in the chapter.

Linearizing the pointing-loss penalty. Expanding the received-power factor Lpoint=exp(−2θerr2/θdiv2) for small jitter gives

Lpoint≈1−2θerrθdiv2,

so holding loss below a budget ϵ requires θerr≤θdivϵ/2. The design implication is direct: tighter, higher-gain beams shrink θdiv and therefore tighten the residual-jitter requirement linearly, which is the quantitative reason the bus alone cannot close the long links and a fine-steering mirror, slaved to the received beacon, is mandatory.

## 14. Communications: Inter-Satellite and Ground

Abstract. This chapter develops the inter-satellite and ground communications link budgets and shows how the inverse-square behaviour of an optical beam forces the satellites into a tight formation.

Optical-beam divergence: the fraction of light captured by the receiver falls as one over range squared, which forces close spacing.

Sources: the optical link budget follows Hemmati [30] and the free-space optical satellite-link analysis of Liang et al. [6].

The communications subsystem sets both the internal fabric capacity and the ability to deliver data to Earth.

Link budget. Telescope gain and free-space loss are

G=πDλ2,  Lfs=4πRλ2,

and the received power in decibels is Prx=Ptx+Gtx+Grx−Lfs−Lpoint−Latm−Loptics.

Why the formation is tight. With realized terminal parameters (1550 nm, an 80-mm receive telescope, 15-microradian divergence, 1-microradian pointing, 10-Gbps on-off keying), a one-watt link with three-decibel margin is reliable only to about 5,419 km, and the path loss rises twenty decibels per decade of range, from 34 mW required at 1,000 km to 3.41 W at 10,000 km, against a commercial terminal limit near one watt. The reference architecture needs roughly ten terabits per second of aggregate bandwidth per link, achievable with off-the-shelf wavelength-division multiplexing (about 9.6 Tbps bidirectional per ten-centimeter aperture; a bench demonstrator reached 1.6 Tbps) only by flying the satellites about 150 m apart. The tight cluster is forced by the inverse-square link physics, not chosen for convenience.

Fig. 14.2 Transmit power required to hold a three-decibel link margin versus range, against the one-watt terminal limit, and the pointing-loss sensitivity to jitter. Range and pointing together force the tight formation.

Ground segment. The downlink to Earth is the availability bottleneck. The slant atmospheric loss scales with the air mass 1/sin(elevation), and cloud cover is opaque at optical wavelengths, so single-site availability is poor. The remedy is site diversity: with n independent ground stations the probability that at least one is cloud-free is 1−(1−pclear)n, which reaches roughly ninety-six percent for four stations. Contact time per overhead pass follows from the elevation-mask geometry, λmax=arccos[(RE/(RE+h))cosε]−ε, giving about nine minutes at a ten-degree mask; combined with the data rate and the number of passes, this sets the daily deliverable volume (tens of terabytes per day in the reference case).

Latency. Light travels at c in vacuum but at c/nfiber in fiber (nfiber=1.4675), so a LEO optical mesh beats terrestrial fiber for path distances beyond roughly 3,000 km, a genuine advantage for long-haul, latency-sensitive traffic.

Fig. 14.1, Inter-satellite free-space loss versus range and pointing-loss sensitivity.

Deriving the free-space loss. The free-space loss in Eq. (14.1) follows from the spreading of an isotropically referenced wave. A transmitter radiating power Ptx uniformly over a sphere of radius R produces an intensity Ptx/(4πR2) at the receiver. An ideal isotropic receive antenna presents an effective aperture Aeff=λ2/(4π), so the captured power between two isotropic terminals is

Prx=Ptx4πR2⋅λ24π=Ptxλ4πR2.

The reciprocal of the bracketed factor is the free-space loss, Lfs=(4πR/λ)2, which recovers the second relation of Eq. (14.1) and makes the inverse-square dependence on range explicit. Expressing this in decibels and differentiating with respect to log10R,

Lfs[dB]=20log10​4πRλ,  d Lfs[dB]d log10R=20,

which is the twenty-decibel-per-decade slope quoted in the chapter.

Gain and beam divergence. A circular aperture of diameter D has on-axis gain G=(πD/λ)2, and its diffraction-limited far-field divergence half-angle scales as θ≈λ/(πw0) for a beam waist w0 comparable to the aperture. The peak intensity therefore concentrates within a solid angle of order θ2, and the link improves as the product GtxGrx partly offsets Lfs.

Pointing loss. A Gaussian beam of full divergence θdiv mispointed by an angle θe suffers the loss

Lpoint=exp​− 2θeθdiv2.

With the chapter values θe=1 μrad and θdiv=15 μrad, the exponent is −2(1/15)2≈−8.9×10−3, a pointing loss near 0.04 dB; the quadratic sensitivity in Eq. (14.4) shows why jitter approaching the divergence angle degrades the budget sharply.

Latency crossover. A path of length d costs time tfiber=nfiber d/c in fiber versus tvac=d/c across a vacuum mesh, so the optical mesh saves Δt=(nfiber−1) d/c per unit path, a fixed fractional advantage of about forty-seven percent that grows linearly with distance and dominates beyond the few-thousand-kilometre scale noted in the chapter.

Design implication. Because loss climbs twenty decibels per decade while pointing loss grows quadratically with jitter, link closure at a one-watt terminal is won far more cheaply by shortening range than by raising power, which is the physical mandate for the tight formation.

## 15. Propulsion and End-of-Life Disposal

Abstract. This chapter sizes the propulsion subsystem and the controlled end-of-life disposal, and shows that the de-orbit maneuver dominates the mission velocity budget.

Controlled de-orbit: a retrograde burn drops the far side of the orbit onto an atmosphere-grazing transfer ellipse.

Sources: the rocket equation and propulsion sizing from Sutton and Biblarz [31]; the Hohmann de-orbit from Curtis [16]; the disposal requirement from FCC 22-74 [7].

Propulsion must make up drag, maintain the formation, enable collision avoidance, and, decisively, de-orbit the satellite within the regulatory window.

The disposal rule. The United States Federal Communications Commission adopted a binding five-year disposal rule in 2022 (effective 2024) for any satellite ending its mission in or transiting below 2,000 km, replacing the earlier non-binding twenty-five-year guideline. Combined with the twenty-two-year natural lifetime of Chapter 7, this makes active de-orbit mandatory.

Controlled de-orbit by a Hohmann burn. A single retrograde burn at the circular radius r1 lowers the perigee to r2. The transfer ellipse has semi-major axis at=(r1+r2)/2, and by vis-viva the speed at its apogee (where the burn occurs) is vapo=vc12r2/(r1+r2), so the required impulse is

Δvdeorbit=vc11−2r2r1+r2.

Worked example: from 650 km to a 180-km perigee, r1=7.021×106, r2=6.551×106, vc1=7534.8 m/s, giving Δvdeorbit=131.6 m/s. This single term dominates the five-year budget of about 190 m/s (drag make-up ~13, formation ~8, collision avoidance ~5, de-orbit ~132, plus margin).

Propellant. The rocket equation, derived from momentum conservation m dv=−ve dm with ve=Ispg0, gives mp=mdry(eΔv/(Ispg0)−1). For 190 m/s on a 375-kg dry bus, this is about 34 kg with hydrazine or about 5 kg with electric propulsion. Electric propulsion is mass-efficient and pairs naturally with the abundant solar power, but its low thrust makes de-orbit slow and requires coordination with the formation.

Fig. 15.1, Per-satellite Δv budget (de-orbit dominates) and propellant mass by propulsion type.

Deriving the de-orbit impulse from vis-viva. The de-orbit relation of equation (15.1) follows from the vis-viva equation, which states that for any conic orbit of semi-major axis a the speed at radius r satisfies

v2=μ2r−1a,

where μ is the gravitational parameter of the Earth. On the initial circular orbit, a=r1, so setting r=r1 gives the circular speed vc1=μ/r1. The retrograde burn places the satellite onto a transfer ellipse whose apogee touches r1 and whose perigee touches r2, hence at=(r1+r2)/2. Evaluating equation (15.2) at r=r1 on this ellipse,

vapo2=μ2r1−2r1+r2=μr1⋅2r2r1+r2,

so that vapo=vc12r2/(r1+r2). The burn need only remove the difference between the circular speed and this apogee speed, and subtracting recovers Δvdeorbit=vc1−vapo, which is equation (15.1).

Deriving the propellant mass from momentum conservation. Integrating the momentum balance m dv=−ve dm from the initial wet mass m0=mdry+mp down to the final dry mass mdry gives

0Δvdv=−vem0mdrydmm ⇒ Δv=veln​m0mdry,

the Tsiolkovsky rocket equation. Writing the mass ratio as m0/mdry=1+mp/mdry and solving for the propellant,

mp=mdryeΔv/(Ispg0)−1,

which is the sizing relation quoted in the chapter. The exponential dependence on Δv/(Ispg0) explains why a high specific impulse is decisive: for the total budget of 190 m/s on the 375-kg dry bus, a hydrazine system at modest Isp needs about 34 kg of propellant, whereas an electric system, with an order-of-magnitude larger Isp, drives the exponent close to zero so that mp≈mdry Δv/(Ispg0) and the requirement falls to roughly 5 kg.

Design implication. Because the de-orbit term dominates the velocity budget and propellant scales exponentially with it, the disposal maneuver, not station-keeping, sets the propulsion architecture; selecting high specific impulse minimizes wet mass at the cost of de-orbit duration, which must be reconciled with the regulatory window and with formation safety.

## 16. Reliability and Constellation Availability

Abstract. This chapter develops reliability and constellation availability: exponential survival, k-of-n redundancy, and the continuous replenishment needed to hold capacity over the mission.

Redundancy and replenishment: the constellation deploys n units and requires k of them, while continuous resupply at about one over the design life per year holds capacity.

Sources: reliability and redundancy methods from the SMAD systems texts [18, 19].

Because no satellite can be repaired in orbit, reliability must be designed in and availability bought through redundancy and replenishment.

Exponential reliability. Let the hazard rate λ(t) be the instantaneous failure rate of a surviving unit. The survival probability obeys dR/dt=−λ(t) R, whose solution is R(t)=exp[−0tλ(t′) dt′]. In the flat region of the bathtub curve the hazard is constant, λ=1/MTBF, so

R(t)=e−λt=e−t/MTBF.

This is the memoryless limit: a constant hazard is exactly the condition under which failures form a Poisson process, and the mean of the resulting exponential lifetime distribution is 1/λ, which is why the mean time between failures appears directly in the exponent. A bus with a mean time between failures of 50,000 hours has R(5 yr)=e−0.876=0.42, so fewer than half survive un-aided; a higher-grade bus at 150,000 hours reaches R(5 yr)=0.75. Either way a bare unit is not enough, and redundancy is mandatory.

k-of-n redundancy. Consider n identical units that fail independently, each surviving with probability p=R(t). The number that survive is binomial, Pr[X=i]=nipi(1−p)n−i. A system that functions when at least k of the n survive therefore has reliability

Rsys=Pr[X≥k]=i=knni pi(1−p)n−i.

The same expression sizes chip-level redundancy within a satellite and satellite-level over-provisioning within a cluster. Its power is in the steep climb of the tail sum: with n=81 nodes required to deliver k=70, a per-node reliability of p=0.90 yields Rsys=0.89, but raising the per-node figure to p=0.95 pushes it to 0.999. Small gains in unit reliability buy large gains in system reliability once the redundancy margin is set.

Constellation availability and replenishment. The expected fraction of required capacity available, absent replenishment, is min(1, ndeployed p/nrequired). For 81 deployed, 75 required, and p=0.90, this is 0.97, so a six-unit margin holds capacity near full even before any resupply. To sustain that level over the mission, units must be replaced as they fail. Modeling the fleet as a renewal process, each unit that fails is replaced after a lead time, and in steady state the replenishment rate equals the failure rate. For a design life L the long-run annual replacement rate is approximately 1/L, about twenty percent per year for a five-year life, and holding a fixed availability target against a given per-unit reliability sets the deployed count: reaching ninety-five percent availability of 75 required nodes at p=0.90 calls for on the order of 89 deployed. The associated replenishment is a major and recurring operating expense, carried explicitly in the economics of Chapter 21. These figures are produced by orbital_dc.reliability (k_of_n, expected_capacity_fraction, spares_for_availability, annual_replacement_rate).

Fig. 16.1, Per-node reliability versus time and the over-provisioning needed to hold capacity.

Derivation of the survival law. The constant-hazard reliability of Eq. (16.1) follows directly from the defining differential equation. Separating variables in dR/dt=−λR gives dR/R=−λ dt, and integrating from the initial condition R(0)=1 yields lnR(t)−lnR(0)=−λt, hence R(t)=e−λt. The probability density of the failure time is the negative derivative of the survivor function,

f(t)=−dRdt=λe−λt,

and the mean lifetime is its first moment,

MTBF=0∞t f(t) dt=0∞λt e−λt dt=1λ.

This integration by parts is the formal reason the mean time between failures appears in the exponent of Eq. (16.1) as 1/λ.

Mean value of the tail sum. The k-of-n reliability of Eq. (16.2) sums a binomial distribution whose expected number of survivors is

E[X]=n p,

with variance np(1−p). The deployed count n must therefore exceed the required count by enough margin that E[X] sits several standard deviations above k; this is the quantitative content of the steep tail-sum climb noted above.

Replenishment as a renewal balance. In steady state the number of in-service units obeys a flow balance: the failure outflow equals the resupply inflow. With N units each failing at rate λ≈1/L, the long-run annual replacement count is

Nrepl=N λ=NL.

For a constellation of 81 units at a five-year design life, Nrepl=81/5≈16 units per year, consistent with the roughly twenty percent annual rate stated above. The deployed count then follows from inverting the availability constraint. A mean-capacity lower bound, holding the expected number of survivors at nrequired for per-unit reliability p and availability target A, is

ndeployed≳A nrequiredp,

which for the figures above gives about 80. The stricter requirement, that the probability of at least nrequired survivors itself reach A evaluated from the binomial tail (16.2), is what spares_for_availability applies and raises the figure to about 89, the value quoted earlier. The design implication is that survival reliability, redundancy margin, and resupply cadence are not independent choices; fixing any two through Eqs. (16.6) and (16.7) determines the third and, with it, the recurring launch and manufacturing budget carried in the mission economics.

## 17. Compute and Workload

Abstract. This chapter relates peak hardware capability to the compute that is actually delivered, after utilization, the radiation tax, and thermal throttling are accounted for.

Delivered compute as a fraction of the headline peak, after real utilization and the radiation tax.

Sources: accelerator throughput and power from vendor documentation [12]; the workload and utilization model from the reference architecture [1].

The purpose of the system is useful computation, and the appropriate figure of merit is delivered useful work, not peak floating-point capability.

Delivered compute. With n chips of peak rate Ppeak, a model-FLOPs utilization MFU, and a radiation availability ar, the delivered useful rate is n Ppeak MFU ar. Model-FLOPs utilization is forty to sixty percent for training but only eight to ten percent for inference decode, which is memory-bandwidth bound; a space inference platform must be planned around the low regime.

Coupled thermal-throttle simulation. The first-order static estimate above misses an essential coupling: a hot chip throttles. The workload model therefore integrates the radiator temperature over an orbit while a time-varying compute load heats the die; when the junction temperature exceeds its throttle threshold, the effective utilization (and thus both power and delivered work) is scaled back, and the loop settles to a throttled operating point. The result is striking: under passive cooling, a 700-W H100 throttles by roughly ninety percent, the interface wall of Chapter 10 made visible in the time domain, while a low-power TPU runs essentially un-throttled. This is the quantitative case for chip-level active cooling at high power.

Effective efficiency. The appropriate efficiency metric is an effective power-usage effectiveness that folds in the radiation throughput tax, PUEeff=(Ptotal/Pcompute)/ar, and the relevant cost metric is dollars per useful petaflop-hour, which at realistic utilization is several times the headline peak figure.

Fig. 17.1, Added subsystems: passive thermal-throttle by chip, ground-station site diversity, latency crossover, and the discounted-cash-flow breakeven.

Decomposing the delivered-compute product. The delivered useful rate is a product of independent derating factors applied to an installed peak. Beginning from n accelerators each rated at peak rate Ppeak, the installed capability is n Ppeak, and each subsequent factor multiplies a fraction in [0,1] onto it. Model-FLOPs utilization MFU removes the work lost to memory stalls and kernel inefficiency; radiation availability ar removes the wall-clock fraction lost to single-event upsets and scrubbing. Collecting the factors gives

Ruseful=n Ppeak MFU ar.

Because the factors enter multiplicatively, their fractional effects add to first order. Writing MFU=1−ℓu and ar=1−ℓr with small loss fractions ℓu and ℓr, expansion of the product yields

Ruseful≈n Ppeak 1−ℓu−ℓr,

so that the throughput penalties combine by simple addition when each is small, and the largest single loss term dominates the design conversation.

The throttled fixed point. The coupled thermal-throttle behavior follows from the steady state where heat generated equals heat rejected. Let dissipation scale with active utilization, P=Ppeak u. When the junction temperature Tj rises above a throttle threshold Tth, firmware reduces u in proportion to the overshoot. A compact model of this control law is

ueff=u0 1−β (Tj−Tth)+,

where u0 is the requested utilization, β is the throttle gain, and ( ⋅ )+ denotes the positive part. The die temperature in turn tracks the dissipated power through the junction-to-radiator thermal resistance Rθ,

Tj=Trad+Rθ Ppeak ueff.

Substituting (17.4) into (17.3) and solving the resulting self-consistent equation for the throttled operating point gives, in the throttling regime,

ueff=u0 1−β (Trad−Tth)1+u0 β Rθ Ppeak.

The denominator shows why a high-power part suffers: the penalty grows with the product β Rθ Ppeak, so a 700-W accelerator on a high-resistance passive path collapses to a small fraction of its requested utilization, while a low-power part with the same β and Rθ remains near u0. This is the quantitative case for chip-level active cooling, which acts on (17.5) by lowering Rθ.

Effective efficiency and unit cost. The effective power-usage effectiveness folds the radiation throughput tax into the conventional overhead ratio,

PUEeff=Ptotal/Pcomputear,

and the unit cost of delivered work follows by dividing the amortized power-and-capital rate by the useful throughput of (17.1),

Cuseful=Craten Ppeak MFU ar.

Equation (17.7) makes the inverse dependence explicit: because MFU for inference decode sits in the eight to ten percent regime quoted in this chapter, the cost per useful petaflop-hour is roughly an order of magnitude above the naive peak figure, before any further radiation derating.

Worked example. Take the inference regime with MFU=0.10 and suppose a radiation availability ar=0.95. Equation (17.1) gives a delivered fraction of 0.10×0.95=0.095, so the platform yields about 9.5 percent of its installed peak. By (17.6), a compute-direct overhead ratio of 1.2 becomes an effective figure of 1.2/0.95≈1.26. By (17.7), the unit cost is inflated by the reciprocal 1/0.095≈10.5 relative to a peak-rated quote.

Design implication. Size the platform on the delivered product of (17.1), not on installed peak, and attack whichever single factor is largest; for an inference-dominated orbital data center that factor is utilization, so memory-bandwidth provisioning and active cooling that lowers Rθ in (17.5) return more delivered work per dollar than adding raw peak silicon.

# PART IV, INTEGRATION, SIMULATION, AND RESULTS

## 18. The Integrated System Model

Abstract. This chapter closes the integrated system: mass, power, thermal, and velocity budgets are solved self-consistently for the reference design point.

The integrated design loop: power, array area, mass, propulsion, and thermal are solved together and iterated to a self-consistent design point.

Sources: the mass, power, thermal, and delta-v closure follows standard concurrent-design practice (SMAD [18]).

The value of a systems model lies in closure: the subsystems must be solved together so that their mutual dependencies are satisfied. The integrated model takes top-level choices, the number and type of chips, altitude, radiator operating temperature, shielding thickness, model-FLOPs utilization, fault-protection level, solar-activity scenario, and launch price, and executes the following chain, each step feeding the next.

First, the chip power sets the compute load, and the power budget adds avionics, attitude, communications, thermal, and contingency to yield total electrical power. That power sizes the solar array area and mass and the battery. The thermal load sizes the radiator area and mass by equation (10.3). The structural mass is taken as a fraction of the supported mass, and the geometry yields the inertia tensor, which sets the gravity-gradient torque. The radiation dose sets the shielding mass. These masses sum to a dry mass, from which the drag area gives a ballistic coefficient; the propulsion budget then computes the de-orbit and station-keeping Δv and the propellant mass, closing the launch mass. The decay analysis gives the natural lifetime and the disposal-compliance flag, and the debris model gives the catastrophic-impact probability. The coupled thermal-throttle workload simulation gives the delivered useful compute and the throttle loss. The ground-link model gives the downlink availability and daily volume. Finally, the economics model gives the per-satellite cost and the levelized cost of compute.

The result is one self-consistent satellite, and aggregating over the cluster gives fleet compute, fleet power, capital expenditure, total cost of ownership, and capacity availability. The mass budget for the reference four-chip satellite is shown in Fig. 18.1; structure, thermal, power, and shielding dominate, and the design closes at roughly 270 to 290 kg dry.

Fig. 18.1, Integrated dry-mass budget, closed self-consistently by the system model.

The closure relations in symbolic form. The integrated model is a fixed-point problem in the satellite mass. To see why, write the dry mass as the sum of the subsystem masses, several of which themselves scale with the total mass through fractional allowances. Let Pc be the chip compute power for a satellite carrying Nc chips, each drawing pc, so that Pc=Nc pc. The total electrical power follows by adding the support-load fractions and a contingency margin,

Ptot=Pc+Pav+Patt+Pcomm+Pth1+fcont,

where fcont is the power contingency. The solar array must supply Ptot at end of life through the eclipse-and-charge duty cycle, so its area is

Asa=Ptotηsa S cosθ (1−ddeg),

with ηsa the cell efficiency, S the solar constant, θ the mean incidence angle, and ddeg the lifetime degradation. The array mass is then msa=ρsaAsa for an areal density ρsa.

The mass fixed point. Group the masses that are fixed by physics (compute, thermal, power, shielding, propellant) into a payload-plus-bus term m0, and let the structure scale as a fraction fstr of the mass it supports. The dry mass then satisfies

mdry=m0+fstr mdry,

which rearranges to the closed form

mdry=m01−fstr.

The factor 1/(1−fstr) is the structural amplification of every kilogram of payload, and it diverges as fstr→1, which is why the structural fraction must be controlled early in the design.

Ballistic coefficient and launch mass. Once mdry is known, the drag area Ad and drag coefficient CD give the ballistic coefficient

β=mdryCD Ad,

a higher β meaning slower decay. The propellant for the total mission Δv follows from the rocket equation, and adding it closes the launch mass,

mwet=mdry exp​ΔvIsp g0,

so the wet mass is the dry mass amplified by the propulsive factor.

Worked example. Take a structural fraction fstr=0.20. Equation (18.4) then gives an amplification 1/(1−0.20)=1.25, so a payload-and-bus term of m0≈224 kg closes at mdry≈280 kg, inside the stated 270 to 290 kg range for the reference four-chip satellite.

Design implication. Because the structural and contingency fractions multiply the entire budget, the cheapest mass savings come from lowering those fractions rather than from trimming any single subsystem; the model exposes this leverage directly through the 1/(1−fstr) amplifier.

## 18A. Computational Methods and Numerical Solvers

Abstract. This chapter documents the numerical methods, root-finding and time integration, used to solve the governing equations, and the checks that anchor them to known results.

Sources: the numerical methods (Newton-Raphson, bisection, Brent, Runge-Kutta) are standard; the orbital integrations are checked against Vallado [17].

The governing equations are not only evaluated in closed form; each is solved with an explicit numerical method in an open solver suite. The quartic radiator energy balance is solved by Newton-Raphson. The drag-decay and eclipse-transient differential equations are integrated by a fourth-order Runge-Kutta scheme. The optical-link range and the internal rate of return are found by Brent root-finding. The launch-cost-parity year is obtained by Monte-Carlo over the uncertain assumptions, because that forecast is skewed and a single point would misrepresent it. Solving the reference mission end to end produces one consistent solution set.

| Quantity | Numerical method | Solution |
| --- | --- | --- |
| Radiator temperature | Newton-Raphson (4 iterations) | 21.3 C |
| Natural lifetime at 650 km | Runge-Kutta decay integration | 19.6 yr |
| Eclipse temperature drop | Runge-Kutta transient | minus 0.6 C |
| Controlled de-orbit delta-v | Hohmann (vis-viva) | 131.6 m/s |
| Optical link range, 3 dB then 0 dB margin | Brent root | 5,419 then 7,655 km |
| Launch-cost parity year | Monte-Carlo | mid-2040s for the minority of scenarios that reach 200 dollars per kilogram within the horizon |

The numerical primitives are general-purpose and documented. The Newton-Raphson core, used for the radiator quartic, is

def newton_raphson(f, fprime, x0, tol=1e-8, max_iter=100):
    x = float(x0)
    for k in range(1, max_iter + 1):
        fx = f(x)
        if abs(fx) < tol:
            return x, k
        x -= fx / fprime(x)
    return x, max_iter

and the fourth-order Runge-Kutta integrator, used for orbital decay and the eclipse transient, is

def rk4_integrate(rhs, y0, t0, t1, dt, stop=None):
    t, y, ts, ys = t0, float(y0), [t0], [float(y0)]
    for _ in range(int((t1 - t0) / dt)):
        k1 = rhs(t, y); k2 = rhs(t + dt/2, y + dt/2*k1)
        k3 = rhs(t + dt/2, y + dt/2*k2); k4 = rhs(t + dt, y + dt*k3)
        y += dt/6 * (k1 + 2*k2 + 2*k3 + k4); t += dt
        ts.append(t); ys.append(y)
        if stop and stop(t, y): break
    return ts, ys

Each solver is checked against the worked examples of this document, so the numbers in the tables and the figures are produced by the same code that a reader can run.

Convergence of the Newton-Raphson radiator solve. The radiator energy balance equates the dissipated electrical load per unit area to the net radiated flux, so the residual whose root is sought has the quartic form

f(T)=εσT4−Tsink4−q=0,

where q is the areal heat load, ε the emissivity, σ the Stefan-Boltzmann constant, and Tsink the effective radiative sink temperature. The Newton-Raphson update follows from the first-order Taylor expansion f(Tn+1)≈f(Tn)+f′(Tn) (Tn+1−Tn)=0, which on setting the left side to zero gives

Tn+1=Tn−f(Tn)f′(Tn)=Tn−εσ(Tn4−Tsink4)−q4 εσ Tn3.

Because f(T) is smooth and monotone in the physical branch T>0, the iteration is locally quadratic: writing the error as en=Tn−T⋆ about the true root T⋆, a second-order expansion of f yields

en+1=f″(T⋆)2 f′(T⋆) en2+O(en3).

The squared error per step is why four iterations suffice to reach the tabulated 21.3 C from a coarse start. As a short worked check, a guess of T0=300 K with the curvature term in (18A.3) reduces an initial error of order 10 K to order 10−1 K after one step and to machine tolerance within the stated four steps, consistent with the tol=10−8 stopping rule of the listed routine.

Local accuracy of the Runge-Kutta integrator. For the scalar decay and eclipse problems y=g(t,y), the classical fourth-order scheme advances the state by the weighted slope average implemented in the listing,

yn+1=yn+h6k1+2k2+2k3+k4,

whose stage combination matches the Taylor series of the exact flow through h4, leaving a local truncation error of order h5 and a global error of order h4,

τn=O(h5),  maxnyn−y(tn)=O(h4).

The fourth-order global scaling means that halving the step h cuts the integration error by roughly sixteen, which is the property that lets the 19.6 yr decay lifetime and the small eclipse temperature drop be resolved without an impractically fine grid. The design implication is direct: solver tolerance and step size, not only the physical inputs, set the precision of every lifetime and thermal-transient figure, so the step must be chosen against the order estimate in (18A.5) before a margin is trusted.

## 19. Uncertainty Quantification and Sensitivity

Abstract. This chapter places uncertainty bands on the headline results by linear error propagation and Monte-Carlo, and identifies the inputs to which the conclusions are most sensitive.

Sources: linear error propagation and Monte-Carlo are standard practice; the input distributions are documented in Appendix C.

A point design is not credible without an uncertainty band. Two complementary tools are used: linear error propagation for a quick analytic estimate, and Monte-Carlo for the full distribution when the inputs span orders of magnitude.

Linear error propagation. For an output y=f(x1,…,xm) with independent inputs of standard deviation σxj, a first-order Taylor expansion about the mean gives

σy2≈j=1m∂f∂xj2σxj2.

This is exact for linear f and a good guide when the inputs vary by a few percent. It fails precisely where the dominant uncertainty here lives, in atmospheric density, which is lognormal and varies by up to two orders of magnitude across the solar cycle, so the partial-derivative picture is supplemented by sampling.

Monte-Carlo propagation. The model draws N samples of the uncertain inputs from their assumed distributions, evaluates the full model for each, and forms the empirical distribution of every output. The dominant inputs sampled are the solar-activity scenario (which moves atmospheric density by up to a factor of one hundred), the debris-flux band, the model-FLOPs utilization, and the undisclosed chip power. The estimator converges as 1/N, so a few thousand samples fix the central percentiles to better than the input uncertainty warrants. The resulting distributions of natural lifetime, catastrophic-impact probability, total Δv, and delivered compute are reported as fifth, fiftieth, and ninety-fifth percentiles rather than as single numbers (Fig. 19.1), produced by orbital_dc.montecarlo.monte_carlo.

Sensitivity ranking. To attribute output variance to inputs, the model uses a one-at-a-time sweep (montecarlo.one_at_a_time) that holds all but one input at its central value and records the output swing. The ratio of that swing to the baseline is a transparent local sensitivity index; where inputs interact, the full Monte-Carlo variance is the honest measure. Two sensitivities stand out. First, the natural lifetime is wildly sensitive to the solar cycle: the same satellite that decays in four years at solar maximum lasts over three hundred years at solar minimum, which is precisely why the design cannot rely on passive disposal. Second, delivered compute is sensitive to model-FLOPs utilization and to the radiation throughput tax, which is why the realistic cost of compute is several times the naive peak-based figure. The single weakest input is the chip power itself, undisclosed by the vendor and treated as a 150-to-200-watt band.

Fig. 19.1, Monte-Carlo distributions of natural lifetime, debris probability, and total Δv across the dominant uncertainties.

Derivation of the linear-propagation formula. Equation (19.1) follows from a first-order Taylor expansion of the output about the input means x=(x1,…,xm). Writing δxj=xj−xj,

y≈f(x)+j=1m∂f∂xj|x δxj.

The mean of y is f(x) to this order, so the deviation is the sum on the right. Forming the variance and using Var(y)=E[(y−y)2] gives, for the general correlated case,

σy2≈j=1mk=1m∂f∂xj∂f∂xk Cov(xj,xk).

When the inputs are independent, the cross terms vanish because Cov(xj,xk)=σxj2 δjk, and (19.3) collapses to (19.1). This makes explicit that (19.1) is the independent-input special case, and that correlated inputs require the full covariance sum.

Relative-error form for products. Many of the chapter’s outputs are products and quotients of inputs, for example a flux multiplied by an area and a time. For a pure power-law monomial y=c j​xjaj, the partial derivative is ∂y/∂xj=aj y/xj, so substitution into (19.1) yields a clean relative-error rule,

σyy2≈j=1maj2σxjxj2.

Each fractional input uncertainty enters weighted by the square of its exponent, so a variable that appears linearly (aj=1) contributes its fractional spread directly, while one entering quadratically contributes twice as strongly.

Worked example. Take the catastrophic-impact probability as a product of debris flux and exposed area over the mission, all with exponent unity. With the chip-power band of 150 to 200 watts, the symmetric fractional spread about the 175-watt midpoint is 25/175≈0.143. If a second independent input carries a comparable 0.143 fractional spread, then by (19.4) the output fractional uncertainty is 0.1432+0.1432≈0.20, about twenty percent, before the lognormal density term is added by sampling.

Design implication. Because uncertainties add in quadrature, the output band is set almost entirely by the one or two widest inputs; tightening the dominant solar-activity and utilization terms narrows the headline bands far more than refining any well-known parameter.

## 20. Results: Reference Design Point and Constellation

Abstract. This chapter collects the reference design-point and constellation results, the numbers that the rest of the book derives and the open model reproduces.

Bringing the chapters together, the reference design point and its key results are summarized in Table 20.1 and the supporting figures.

| Quantity | Result | Driver / chapter |
| --- | --- | --- |
| Orbital period | 97.6 min | Ch. 4 |
| Natural lifetime (moderate solar) | ~22 yr | Ch. 7, fails 5-yr rule |
| Radiator area (1 MW, 20 C) | ~1,508 m² | Ch. 10 |
| Interface ΔT (700 W chip) | 210 C (>125 budget) | Ch. 10, active cooling needed |
| 5-yr dose behind 10 mm Al | ~1.06 krad(Si) | Ch. 8, large margin |
| Catastrophic debris P (cluster, 5 yr) | ~17% (6-46% band) | Ch. 9 |
| In-cluster cascade P (neighbor, 150 m) | ~8.5% | Ch. 13 / risk synthesis |
| De-orbit Δv | 131.6 m/s (of ~190 total) | Ch. 15 |
| Passive H100 throttle loss | ~90% | Ch. 17 |
| Reference dry mass | ~270-290 kg | Ch. 12, 18 |

The debris and radiation results, central to the survivability case, are shown in Figs. 20.1 and 20.2. The debris figure shows both the cluster-level catastrophic probability over the mission and the in-cluster cascade coupling versus formation spacing; the radiation figure shows the dose-versus-shielding curve against the memory tolerance and the reliability-versus-throughput trade.

Fig. 20.1, Catastrophic debris risk for the full cluster, and the in-cluster cascade coupling versus formation spacing.

Fig. 20.2, Cumulative dose versus aluminium shielding, and the reliability-throughput trade.

From orbital period to the survivability clock. The reference period in Table 20.1 follows directly from Kepler’s third law for a near-circular orbit. With μ the standard gravitational parameter of Earth, R⊕ the mean radius, and h the orbital altitude, the semi-major axis is a=R⊕+h and the period is

T=2πa3μ=2π(R⊕+h)3μ.

Differentiating gives the sensitivity of the period to altitude, which sets how tightly a constellation must hold its altitude band before its members drift in phase:

dTdh=3πμ R⊕+h.

The orbital mean motion n=2π/T then fixes the revisit and eclipse cadence that the thermal and power budgets of the preceding chapters are built upon.

Why the lifetime fails the five-year rule. The natural-lifetime entry is governed by atmospheric drag removing orbital energy. For a circular orbit the specific energy is ε=−μ/(2a), so a small decay in semi-major axis costs energy at the rate

dεdt=μ2a2 dadt,

and the drag force on a body of mass m, cross-section A, and drag coefficient CD in atmospheric density ρ moving at v=μ/a does work at the rate that drains this energy. Balancing the two yields the decay rate of the semi-major axis,

dadt=− CD Am ρ μ a.

The grouping A/m is the area-to-mass ratio, the single most important design variable here. A compact dry mass of roughly 270 to 290 kg, as listed in Table 20.1, drives A/m low, which lengthens the lifetime toward the cited 22 years and is precisely why the design fails the 5-year passive-disposal rule and must carry the de-orbit budget of Chapter 15.

Worked example, debris-risk scaling. The catastrophic-collision probability over a mission accumulates as a Poisson process with hazard rate proportional to the swept cross-section. If a single satellite carries probability p1, then for a cluster of N effectively independent members the cluster-level probability is

Pcluster=1−(1−p1)N.

For small p1 this is close to Np1, which is an upper bound: the exact cluster probability 1−(1−p1)N is concave in N and bends below the linear estimate, saturating toward one. The cluster figure of about 17 percent is therefore dominated by member count, and adding satellites raises the cluster risk close to linearly at this scale, with the per-member marginal risk slowly decreasing as N grows. The design implication is direct: constellation sizing cannot be separated from the per-member debris hazard, and growth in N must be paid for in either shielding mass or accepted mission risk.

# PART V, ECONOMICS AND PREDICTIONS

## 21. Techno-Economics: Cost, NPV, and the Levelized Cost of Compute

Abstract. This chapter develops the techno-economics, capital and operating cost, net present value, and the levelized cost of useful compute.

Sources: the levelized-cost and net-present-value formulation follows standard engineering economics and the SMAD cost treatment [18]; launch-cost inputs per [35] and the reference architecture [1].

The economic case is built with a proper time-value-of-money treatment rather than simple cost bookkeeping.

Cost structure and the launch cross-over. Per-satellite cost is launch plus hardware, Csat=plaunch mlaunch+Chw. Launch equals hardware at the cross-over price p*=Chw/mlaunch; with about 1.1 million dollars of hardware on a 415-kg satellite, p*≈2,650 USD/kg. Below this price, hardware dominates, and below roughly 600 USD/kg the launch term is a small correction. The economics are therefore gated by hardware cost, lifetime, utilization, and the radiation tax, not by the headline dollar-per-kilogram figure that dominates public discussion.

Discounted cash flow. The capital-recovery factor that annualizes a capital cost over n years at discount rate r is CRF=r(1+r)n/[(1+r)n−1]. Net present value is NPV=t​CFt/(1+r)t, and the internal rate of return is the rate at which NPV vanishes, solved by bisection.

Levelized cost of compute. The most decision-relevant metric is the levelized cost per useful petaflop-hour:

LCOE=CRF⋅CAPEX+OPEXyrannual useful PFLOP-hours.

Because the denominator uses delivered, throttled, utilization-and-availability-derated compute, a high-power chip that throttles under passive cooling shows a dramatically worse levelized cost than its peak rating implies, quantifying, in dollars, the penalty of the interface wall. Replenishment, at the reliability-driven rate of Chapter 16, is a first-order operating expense in this calculation.

Fig. 21.1b Levelized cost of useful compute versus model-FLOPs utilization, and versus launch price. The cost is dominated by utilization and hardware, not by launch.

Fig. 21.1, Cost of useful compute versus utilization, and the radiation throughput tax by protection level.

Deriving the launch cross-over price. The per-satellite cost separates into a launch term that scales with delivered mass and a fixed hardware term,

Csat=plaunch mlaunch+Chw.

The two contributions are equal when plaunch mlaunch=Chw. Solving for the price isolates the cross-over,

p*=Chwmlaunch.

With the chapter’s reference values, Chw≈1.1×106 USD and mlaunch=415 kg, the cross-over evaluates to p*=1.1×106/415≈2,650 USD/kg, consistent with the figure quoted in the body. The derivative ∂Csat/∂plaunch=mlaunch is constant, so once plaunch≪p* the satellite cost flattens onto the hardware floor and further reductions in launch price return little.

The capital-recovery factor as a geometric sum. Annualizing a capital cost requires the level payment A whose discounted stream over n years equals the principal. Writing the present value of n equal payments and summing the geometric series gives

CAPEX=At=1n1(1+r)t=A 1−(1+r)−nr,

and inverting for the annuity per unit principal yields the capital-recovery factor used in (21.1),

CRF=ACAPEX=r (1+r)n(1+r)n−1.

In the limit r→0 the factor reduces to CRF→1/n, recovering straight-line amortization; for r>0 it exceeds 1/n, charging the project for the time value of locked capital.

Useful compute and the utilization chain. The denominator of (21.1) is the peak rating successively derated by the model-FLOPs utilization u, the operational availability Aop, and the throttling factor ηth set by the thermal interface,

PFLOP-hoursyr=Ppeak ηth u Aop (8760 h).

Because LCOE∝1/(ηth u Aop), the levelized cost is hyperbolic in each derating term; a chip that throttles to ηth=0.5 doubles the cost of every delivered petaflop-hour regardless of its peak specification.

Design implication. The economics are governed by hardware cost, lifetime, and the multiplicative utilization chain rather than by the headline launch price, so design effort returns more when spent raising sustained utilization and availability than when spent chasing marginal dollar-per-kilogram gains below the cross-over.

## 22. The Launch-Cost-Parity Prediction

Abstract. This chapter tests the launch-cost-parity assumption that underwrites the business case, and finds that most sampled futures never reach the target price within the horizon, with parity, where it arrives, falling later than the headline date.

Sources: the learning-curve and cadence inputs follow the reference architecture’s cost model [1] and public launch data [35].

The entire economic case rests on launch cost falling far enough that the annualized cost of power in orbit approaches the terrestrial cost of power. The reference paper projects launch reaching 200 dollars per kilogram by about 2035 under a twenty-percent learning rate, requiring on the order of 370,000 tonnes of additional cumulative launched mass, some 1,800 Starship launches, at which point the launched-power price of roughly 810 dollars per kilowatt-year approaches the terrestrial data-center power spend of 570 to 3,000 dollars per kilowatt-year. The broader launch-cost landscape that frames this target is shown in Fig. 22.1: today’s reusable launch still sits an order of magnitude above it, and once the price falls below roughly 600 dollars per kilogram the all-in cost per satellite is set by hardware rather than launch, so further reductions in launch price yield diminishing returns.

Fig. 22.1. The launch-cost landscape. Left: below roughly 600 dollars per kilogram the all-in cost per satellite is dominated by hardware rather than launch. Right: the price-per-kilogram trajectory to low Earth orbit, from the Shuttle program average through reusable Falcon 9 to the realistic and aspirational Starship targets.

This forecast is assumption-fragile, so this study generates its own credibility band rather than inheriting a single date. A Wright learning curve is applied to cumulative launched mass, with a logistic ramp in launch cadence, and the unproven assumptions are sampled by Monte-Carlo: the learning rate from fifteen to twenty-five percent, the 2026 effective price from 1,200 to 3,500 dollars per kilogram, the payload from 150 to 250 tonnes, and the peak cadence from 80 to 220 launches per year. The first result is the more important one and is easy to miss in a single headline date: across the sampled assumptions, only about five percent of scenarios reach 200 dollars per kilogram at all within the 2050 horizon, and roughly ninety-five percent never reach it. Parity at that price is therefore not a near-certainty awaiting a date; under most defensible assumptions it does not arrive within the horizon. Conditional on the minority of scenarios that do reach it, the median parity year is near 2044 (Fig. 22.1), with a fifth percentile of about 2037 and essentially no probability of reaching parity by 2035. Extending the horizon to 2060 raises the reaching fraction only to about nine percent and moves the conditional median to the end of the 2040s, so the conclusion is not an artifact of where the horizon is drawn. The vendor’s 2035 date sits beyond the optimistic edge of the distribution and requires its specific aggressive anchors, full reusability at 200 tonnes and roughly two million dollars marginal cost, sustained twenty-percent learning from a low starting price, and a total launched mass far larger than the cadence ranges sampled here deliver.

The prediction is therefore twofold: launch-cost parity at 200 dollars per kilogram is unlikely under most assumptions rather than merely late, and where it does arrive it is more likely in the late 2030s to mid-2040s than in 2035. Parity is in any case necessary but not sufficient: even at parity, the business case is gated by hardware cost, lifetime, and utilization.

Fig. 22.2, Launch-cost learning curve (Monte-Carlo) and the distribution of the cost-parity year. Most sampled scenarios never reach 200 dollars per kilogram within the horizon; conditional on those that do, the median is near 2044, later than the vendor’s 2035.

The Wright learning curve in detail. The cost model rests on a Wright relation, in which the unit price of launch falls by a constant fraction for every doubling of cumulative launched mass. If C(m) is the effective price per kilogram at cumulative launched mass m, and C0 is the price at a reference mass m0, the relation is

C(m)=C0mm0−b,

where the exponent b encodes the learning rate. The learning rate L is defined as the fractional reduction per doubling, so that two successive doublings satisfy C(2m)/C(m)=1−L. Substituting m→2m into (22.1) gives C(2m)/C(m)=2−b, and equating the two expressions yields

2−b=1−L,  b=−ln(1−L)ln2.

A twenty-percent learning rate, L=0.20, therefore corresponds to b=−ln(0.80)/ln2≈0.322.

Solving for the mass required to hit a target price. The design question is inverted from (22.1): given a target price C*, how much cumulative mass must be launched? Setting C(m)=C* and rearranging,

mm0=C*C0−1/b,

so the required cumulative mass grows as a steep inverse power of the target price. With a target of 200 dollars per kilogram, the small ratio C*/C0 is amplified by the exponent 1/b≈3.1, which is why the model calls for cumulative launched mass on the order of hundreds of thousands of tonnes.

The cadence ramp and the parity year. Cumulative mass accrues over time through flights. With a logistic ramp in annual cadence approaching a peak rate, the cumulative mass at time t is the integral of payload times cadence,

m(t)=m0+pt0tR(τ) dτ,

where p is payload per launch and R(τ) is the launches-per-year ramp. The parity year t* is the time at which m(t*) first equals the mass required by (22.3); combining (22.3) and (22.4) shows that t* depends jointly on the learning exponent, the starting price, the payload, and the integrated cadence, which is why sampling these four inputs spreads parity across a wide band rather than fixing a single date.

Worked estimate. Take L=0.20, so b≈0.322 and 1/b≈3.1. If the effective price must fall by roughly a factor of ten to approach the target, then from (22.3) the cumulative mass must rise by a factor of about 103.1≈1.3×103 over its reference value, which at a payload near 200 tonnes per flight calls for many hundreds of flights even before the slower logistic tail.

Design implication. Because required mass scales as the target price raised to the power −1/b, the parity date is acutely sensitive to the learning rate and the starting price; a planner should treat parity as a distribution conditioned on those inputs, not a fixed milestone, and should not commit capital on the assumption that the headline date will arrive.

## 23. Capability and Risk Trajectory, 2027-2035

Abstract. This chapter synthesizes the preceding analysis into a defensible capability and risk trajectory through the early 2030s, and names the gating demonstrations.

Technology-readiness ladder: the components sit mid-scale, while system integration sits near the bottom.

Synthesizing the technology-readiness picture of Chapter 2 with the engineering constraints of the preceding chapters yields a defensible capability trajectory. By 2027, the demonstration missions (the Suncatcher two-satellite prototype, Starcloud’s Blackwell-class follow-on) should place real accelerators in orbit and exercise components, raising their readiness while leaving constellation-scale operation unproven. Through 2028 to 2030, the gating demonstrations are flown multi-terabit formation links, validated formation station-keeping under the differential drag of the solar cycle, and, most importantly, autonomous, cluster-coordinated debris avoidance at sub-kilometer spacing. From 2030 to 2035, if launch trends hold, niche economic cases emerge for latency-critical or space-native workloads, while gigawatt scale remains aspirational and cost parity, per Chapter 22, is unlikely before the mid-2030s.

The debris-and-Kessler trajectory deserves particular attention: a one-kilometer cluster in the congested 650-km shell raises both the catastrophic-impact exposure and the cascade hazard, and its evolution depends on disposal compliance and avoidance autonomy. A flight program should quantify this with the official ORDEM 3.2 and LEGEND tools before committing to constellation scale.

From component readiness to system readiness. The chapter treats the readiness ladder qualitatively, with components mid-scale and integration near the bottom. A useful way to make this precise for a constellation is to recognize that a serial chain of independently gating subsystems behaves multiplicatively. If the program must clear n gating demonstrations and each demonstration i has an independent success probability pi, then the probability that the integrated system reaches operational readiness is the product

Psys=i=1npi.

Taking logarithms converts the product into a sum that exposes the weakest link,

lnPsys=i=1nlnpi,

so the demonstration with the smallest pi contributes the largest negative term and dominates the schedule. This is the formal reason the chapter elevates autonomous, cluster-coordinated debris avoidance above the link and station-keeping milestones: a single low-probability gate caps the whole product. For a quick worked figure, the three gating demonstrations named in the text, formation links, station-keeping under solar-cycle drag, and avoidance autonomy, with illustrative values p1=0.9, p2=0.8, p3=0.5, give Psys=0.9×0.8×0.5=0.36, confirming that the autonomy gate sets the ceiling.

Cascade exposure in the congested shell. The debris-and-Kessler concern can be carried one step further than the prose. For a cluster presenting collision cross-section A moving at relative speed vrel through a debris number density ρ, the expected catastrophic-impact rate is

λ=ρ A vrel,

and over a mission of duration T the probability of at least one such impact, assuming Poisson statistics, is

Phit=1−e−λT.

Avoidance autonomy enters as a multiplier that suppresses the effective rate to λeff=(1−η) λ, where η is the fraction of conjunctions resolved by maneuver; the cascade hazard scales with the residual λeff together with disposal compliance. These relations are the analytic skeleton that the official ORDEM 3.2 and LEGEND tools populate with shell-specific densities.

Design implication. Schedule and capital should be concentrated on raising the lowest-probability gate, since equations (23.1) and (23.2) show it bounds system readiness; and avoidance autonomy must be sized to drive η high enough that the residual rate in equation (23.3) keeps cascade exposure within the program’s risk budget.

# PART VI, SYNTHESIS

## 24. Caveats and Limitations

Abstract. This chapter states plainly what the study does and does not establish, including the quality of its sources and the model fidelity it deliberately stops short of.

A complete assessment must state clearly what this study does and does not establish.

On source quality: the single most load-bearing source for the radiation, link-budget, and cost-parity claims is a non-peer-reviewed, vendor-authored preprint; its results are self-reported and not independently replicated. On the technology-readiness gap: what has been demonstrated is one data-center-class GPU in orbit, ground radiation testing of a TPU, a bench-scale optical link, and kilowatt-class nodes; what the vision requires, megawatt-to-gigawatt clusters, flown multi-terabit formation links, dramatically cheaper launch, and in-flight formation control, has not been demonstrated. On model fidelity: the debris flux, atmospheric density, and radiative limits used here are computed at first order with explicit uncertainty bands, not measured per-orbit; a flight program would use the official agency tools (NASA DAS, ORDEM 3.2, ESA MASTER and DRAMA, SPENVIS/SHIELDOSE-2) and a full thermal and orbit toolchain. The model also omits, by design, a multi-node thermal finite-element analysis, a closed-loop attitude-control simulation, formal failure-modes and effects analysis, micrometeoroid Whipple-shield sizing, and a full time-domain mission simulation. Finally, several widely repeated marketing claims, notably “ten times cheaper including launch”, failed independent verification and are not relied upon here.

These limitations do not undermine the study’s purpose, which is to size the problem, rank the risks, and quantify the trades transparently. They do bound the confidence with which any single number should be quoted, and they identify exactly where higher-fidelity work is needed next.

On compounding the independent uncertainty bands. The chapter reports several first-order quantities, each carrying an explicit uncertainty band rather than a measured value. When a derived figure of merit depends on more than one of these quantities, the bands must be combined rather than quoted singly. For a quantity Q that is a product of independent inputs xi,

Q=i=1nxi ai,

the relative uncertainty follows from logarithmic differentiation. Taking logarithms gives lnQ=i​ailnxi, and differentiating yields dQ/Q=i​ai dxi/xi. Treating the inputs as independent and adding their variances in quadrature gives

σQQ2=i=1nai2σxixi2.

This relation explains why any single number derived from the model carries a wider band than its individual inputs, and why a low-fidelity quantity propagates into every figure of merit that uses it.

On the dominance of the worst input. Equation (24.1) shows that the term with the largest |ai| σxi/xi governs the total. If one input dominates so strongly that ak2(σxk/xk)2≫i≠k​ai2(σxi/xi)2, then

σQQ ≈ akσxkxk.

For this study the single most load-bearing input is the vendor-authored preprint underlying the radiation, link-budget, and cost-parity claims, so any cost-parity figure of merit inherits a band set chiefly by that one self-reported source.

Worked example. Suppose a cost-parity ratio is formed from three first-order inputs of equal exponent ai=1, with relative bands of 30%, 20%, and 10%. Equation (24.1) gives

σQQ=0.302+0.202+0.102=0.14≈0.37.

The combined band, about 37%, exceeds the largest single band of 30%, and Equation (24.2) recovers that 30% term as the leading contribution.

Design implication. Confidence in any quoted figure should be set by the combined band of Equation (24.1), not by the tightest input; the highest-value next step is to replace the dominant low-fidelity source identified by Equation (24.2) with an independently measured value, since that single substitution shrinks the band more than refining all the smaller terms together.

## 25. Conclusions and the Recommended Demonstrator

Abstract. This chapter draws the conclusions and recommends the program that the analysis supports: small, attritable demonstrators aimed at the binding survivability and autonomy risks.

The analysis supports a clear and defensible conclusion. Radiative cooling, the problem most often assumed to be the obstacle, is the solved part, now flight-proven at the single-node scale, with the caveat that it does not scale for free and that high-power chips require active cooling at the die. Radiation is comfortably survivable with sensible shielding, at the cost of a modest throughput tax. The binding, unsolved constraints are elsewhere: orbital-debris survivability in a tight formation, where the same close spacing that enables terabit optical links makes a fragmentation cascade and collision avoidance dangerous; credible end-of-life disposal, which the 650-km orbit cannot achieve passively within the five-year rule and which therefore mandates propulsion; and economics, where the launch-cost parity that underwrites the business case is later and less certain than advertised.

The recommended program follows directly. It is precisely what the leading efforts are already doing: small, attritable, one-or-two-satellite demonstrators that retire the risks that actually matter. Such a demonstrator should measure in-situ radiation dose and upset rates on the target accelerator, demonstrate a modular liquid-cooled radiator at real kilowatt-class chip power, demonstrate onboard debris sensing and an autonomous avoidance maneuver, and demonstrate a controlled propulsive de-orbit at end of mission. Constellation-scale deployment should be gated behind a demonstrated answer to the formation-and-debris paradox, not behind a launch-cost milestone. The systems-level risk picture is summarized in Fig. 25.1: cooling sits in the low-risk corner, while debris, disposal, and economics sit in the high-risk corner, and that is where the engineering attention belongs.

Fig. 25.1, Systems-level risk matrix: cooling is solved; debris, disposal, and economics are the open items.

The disposal constraint and the propulsion it mandates. The chapter concludes that the 650-km orbit cannot meet the five-year post-mission disposal rule passively, so propulsion is required. To see why a controlled de-orbit burn is the natural remedy, begin from the impulse needed to lower perigee into the dense atmosphere. For a near-circular orbit of radius r about a body of gravitational parameter μ, the circular speed follows from balancing gravity against the centripetal requirement,

μr2=vc2r ⇒ vc=μr.

A single tangential retro-burn places the spacecraft on a transfer ellipse whose apogee is the present radius ra=R⊕+h and whose perigee rp is chosen low enough that drag completes the decay. From the vis-viva relation, v2=μ​2/r−1/a with semi-major axis a=(ra+rp)/2, the speed required at apogee is

va=μ​2ra−2ra+rp,

so the de-orbit impulse is the deficit between the circular speed and this transfer speed,

Δvdeorbit=vc−va=μra−μ​2ra−2ra+rp.

The propellant mass to deliver this impulse is then set by the rocket equation, with m0 the wet mass, mf the dry mass, and Isp the specific impulse,

mpm0=1−exp​−ΔvdeorbitIsp g0.

Worked example. Take μ=3.986×105 km3/s2, R⊕=6378 km, and the chapter’s h=650 km, so ra=7028 km and vc=μ/ra≈7.53 km/s. Targeting a perigee at rp=R⊕+100 km=6478 km gives a=6753 km and va≈7.38 km/s, hence Δvdeorbit≈0.15 km/s. With Isp≈220 s, equation (25.4) yields mp/m0≈1−exp(−150/2158)≈0.067, roughly seven percent of wet mass committed to disposal.

Design implication. A controlled de-orbit from the 650-km orbit costs only a small velocity increment and a single-digit propellant fraction, so the disposal risk is not one of feasibility but of mandating, sizing, and reserving that propulsion within the demonstrator from the outset rather than treating it as optional.

# PART VII, DETAILED RESULTS, DATA, AND WORKED COMPUTATIONS

## 26. A Worked End-to-End Computation

Abstract. This chapter carries a single design point end to end, from orbit through thermal, link, disposal, and compute, so the chain from principle to number can be followed in one place.

To make the integrated model concrete, this chapter walks through the reference design point exactly as the code computes it, carrying the numbers at each step. The inputs are four TPU-class accelerators at an estimated 200 W each, a 650-km dawn-dusk orbit, a radiator operating point of 45 C, 10 mm of aluminium shielding, a model-FLOPs utilization of 0.30, error-correcting memory with checkpointing, moderate solar activity, and a launch price of 300 dollars per kilogram.

The compute load is four times 200, or 800 W. The power budget adds 150 W of avionics, 80 W of attitude control, 50 W of communications, 20 W of thermal control, and a fifteen-percent contingency, giving a total electrical demand of 1,265 W and an electrical power-usage effectiveness of 1.58 (Table T10). That total sizes a solar array of about five square meters and a small dawn-dusk battery. The thermal load, compute plus avionics plus parasitic, sizes a modest radiator by equation (10.3). The radiation dose behind 10 mm of aluminium is about 1.05 krad(Si) over five years, comfortably within the memory tolerance (Table T4). The structural mass is taken as a fifth of the supported mass, and the geometry yields an inertia of a few hundred kilogram-meters squared, from which the gravity-gradient torque is of order one ten-thousandth of a newton-meter. Summing all subsystems gives a dry mass near 220 kg (Table T9), from which the drag area sets a ballistic coefficient that feeds a natural lifetime of roughly twelve to twenty-two years depending on solar activity, failing the five-year rule and mandating the 132 m/s de-orbit burn of Table T7. The coupled thermal-throttle simulation returns the delivered useful compute (about two-thirds of a petaflop for this low-power, lightly throttled case), the ground-link model returns tens of terabytes per day of downlink at ninety-six-percent availability, and the finance model returns a levelized cost of roughly one hundred dollars per useful petaflop-hour (Table T11). Aggregating over the eighty-one-satellite cluster gives the fleet totals of Table T12. Every one of these numbers is produced by the open code and is reproduced by the validation suite.

From compute load to electrical demand. The chapter assembles the bus power by summing the accelerator draw with the fixed subsystem allocations and then applying a contingency factor. Writing PIT for the compute load and {Pi} for the avionics, attitude-control, communications, and thermal-control allocations, the total electrical demand is

Pelec=(1+m)PIT+i​Pi,

where m is the contingency margin. With PIT=4×200=800 W, the subsystem sum 150+80+50+20=300 W, and m=0.15, equation (26.1) gives Pelec=1.15×1100=1,265 W, the figure carried in the text. The electrical power-usage effectiveness follows directly as the ratio of total demand to compute draw,

PUEelec=PelecPIT=1,265800≈1.58,

which reproduces the tabulated value and shows that the non-compute overhead inflates the array and battery sizing by very nearly sixty percent above the bare accelerator load.

Lifetime, ballistic coefficient, and the disposal trigger. The natural orbital lifetime scales inversely with the ballistic coefficient, so the dry-mass and drag-area estimates feed the disposal decision. Defining the ballistic coefficient in the usual form,

β=mdryCDA,

a larger β means a denser, more compact body that lingers longer at altitude. The de-orbit requirement is then governed by a simple inequality between the natural lifetime Tnat(β) and the regulatory limit Trule,

Tnat(β)>Trule ⇒ active disposal required.

For the reference point Tnat falls in the twelve-to-twenty-two-year band while Trule=5 yr, so the inequality holds and the active de-orbit burn of Table T7 is mandated; the impulsive velocity increment of that burn is set by the perigee-lowering maneuver and amounts to the 132 m/s already quoted.

Levelized cost as a normalized figure of merit. The finance result is most useful when read as compute per unit cost. Writing Clife for the lifecycle cost, Q for the delivered useful compute rate, and Top for the operating horizon,

LCC=ClifeQ Top,

so the roughly one hundred dollars per useful petaflop-hour of Table T11 places the dominant leverage on Q, the throttled compute, rather than on launch price alone. Design implication: because PUEelec and the throttled Q enter the cost through (26.2) and (26.5), reducing parasitic overhead and thermal throttling improves the economics more than equivalent reductions in launch price, while the lifetime inequality (26.4) fixes disposal mass and propellant before any of that economics is realized.

## 27. Detailed Results Tables

Abstract. This chapter tabulates the detailed numerical results that support the figures and conclusions of the preceding parts.

The following tables report the model output across the relevant input ranges. They are generated directly by the model (report/gen_tables.py) so that every value is consistent with the equations of Parts II and III.

Table T1, Orbital parameters versus altitude. Speed, period, critical beta angle, and natural lifetime. Note how steeply lifetime rises with altitude, and that 650 km already fails the five-year rule at moderate solar activity.

| Alt (km) | v (km/s) | Period (min) | β* (deg) | Lifetime, moderate (yr) |
| --- | --- | --- | --- | --- |
| 400 | 7.67 | 92.4 | 70.2 | 5 |
| 500 | 7.62 | 94.5 | 68.0 | 7 |
| 600 | 7.56 | 96.5 | 66.1 | 13 |
| 650 | 7.53 | 97.6 | 65.2 | 20 |
| 700 | 7.51 | 98.6 | 64.3 | 32 |
| 800 | 7.46 | 100.7 | 62.7 | 97 |
| 1000 | 7.35 | 105.0 | 59.8 | 309 |

Table T2, Radiator area (m²) versus heat load and operating temperature (double-sided, ε = 0.90, 12 % parasitic). The linearity in load and the benefit of running hot are both visible.

| Heat load | 20 C | 40 C | 60 C |
| --- | --- | --- | --- |
| 1 kW | 2 | 1 | 1 |
| 10 kW | 15 | 12 | 9 |
| 100 kW | 151 | 116 | 90 |
| 1 MW | 1,508 | 1,158 | 904 |

Table T3, Chip-interface temperature rise and passive feasibility (R_th = 0.30 K/W). The interface wall is stark: every GPU-class part exceeds the entire 125 C budget across the chain alone.

| Chip | TDP (W) | Interface ΔT (C) | Passive feasible (<125)? |
| --- | --- | --- | --- |
| TPU v6e | 200 | 60 | yes |
| TPU | 300 | 90 | yes |
| H100 | 700 | 210 | NO |
| B200 | 1000 | 300 | NO |
| GB200 | 1200 | 360 | NO |

Table T4, Cumulative total ionizing dose, rad(Si), versus aluminium shielding. The 10-mm, five-year value of ~1 krad sits comfortably below the 2-krad first-irregularity threshold.

| Al (mm) | 5-yr (rad) | 10-yr (rad) |
| --- | --- | --- |
| 1 | 81,169 | 162,339 |
| 3 | 25,479 | 50,959 |
| 5 | 8,307 | 16,613 |
| 10 | 1,054 | 2,109 |
| 15 | 671 | 1,343 |
| 20 | 651 | 1,302 |

Table T5, Catastrophic (>1 cm) debris probability (%) for the 81-satellite cluster. The low/mid/high columns span the ORDEM/MASTER flux band.

| Years | low flux | mid | high |
| --- | --- | --- | --- |
| 1 | 1.2 | 3.6 | 11.4 |
| 2 | 2.4 | 7.0 | 21.6 |
| 3 | 3.6 | 10.4 | 30.5 |
| 5 | 5.9 | 16.7 | 45.5 |
| 7 | 8.2 | 22.5 | 57.3 |
| 10 | 11.4 | 30.5 | 70.3 |

Table T6, In-cluster cascade neighbor-hit probability (%) versus formation spacing. The inverse-square fall-off is the formation paradox quantified.

| Spacing (m) | P_hit (%) |
| --- | --- |
| 100 | 18.1 |
| 150 | 8.5 |
| 200 | 4.9 |
| 300 | 2.2 |
| 500 | 0.8 |
| 1000 | 0.2 |

Table T7, Per-satellite Δv budget and propellant. De-orbit dominates; electric propulsion is dramatically lighter.

| Δv component | Value (m/s) |
| --- | --- |
| Drag make-up | 14.7 |
| Formation keeping | 8.0 |
| Collision avoidance | 5.0 |
| Controlled de-orbit | 131.6 |
| Margin (20 %) | 31.9 |
| Total | 191.2 |

| Propulsion | Isp (s) | Propellant for total Δv (kg) |
| --- | --- | --- |
| Cold gas | 70 | 120 |
| Hydrazine | 220 | 35 |
| Electric | 1500 | 5 |

Table T8, Optical link budget versus range (D = 0.1 m, λ = 1.55 µm). The +20 dB per decade of range, against a fixed ~106 dB telescope gain, is why the cluster must be tight.

| Range | Telescope gain (dB) | Free-space loss (dB) |
| --- | --- | --- |
| 150 m | 106 | 182 |
| 1 km | 106 | 198 |
| 100 km | 106 | 238 |
| 1000 km | 106 | 258 |
| 10000 km | 106 | 278 |

Table T9, Reference satellite mass budget (four-chip bus, closed by the integrated model).

| Subsystem | Mass (kg) |
| --- | --- |
| Shielding | 54 |
| Avionics | 45 |
| ADCS | 30 |
| Harness | 28 |
| Structure | 27 |
| Power | 18 |
| Propulsion | 13 |
| Payload (chips) | 12 |
| Thermal | 6 |
| Dry total | 220 |
| Launch mass | 233 |

Table T10, Reference power budget. Non-compute overhead dominates a small bus, giving an electrical PUE near 1.6; efficiency improves only at many-chip scale.

| Consumer | Power (W) |
| --- | --- |
| Compute | 800 |
| Avionics | 150 |
| ADCS | 80 |
| Communications | 50 |
| Thermal control | 20 |
| Contingency | 165 |
| Total | 1,265 |
| Electrical PUE | 1.58 |

Table T11, Reference design-point summary (all model outputs).

| Quantity | Value |
| --- | --- |
| Compute power (W) | 800 |
| Total power (W) | 1,265 |
| Effective PUE (per usable compute, incl. radiation derate) | 1.84 |
| Radiator area (m²) | 1.1 |
| 5-yr dose behind 10 mm (rad) | 1,054 |
| Dose margin (vs 2 krad) | 1.9× |
| Natural lifetime (yr) | 11.5 |
| Debris P (5 yr) | 0.17 |
| Delivered (PFLOPS) | 0.67 |
| Throttle loss | 14 % |
| Ground downlink (TB/day) | 26.9 |
| LCOE (USD/PFLOP-hr) | 108 |

Table T12, Constellation summary (81 satellites).

| Quantity | Value |
| --- | --- |
| Fleet compute (PFLOPS) | 54.1 |
| Fleet power (MW) | 0.10 |
| CAPEX (USD) | 146,621,400 |
| 10-yr TCO (USD) | 317,621,400 |
| Capacity availability | 0.89 |

## 28. Constellation Design and Relative Dynamics

Abstract. This chapter treats the relative dynamics of the tight formation, the Clohessy-Wiltshire relations and the station-keeping they imply.

Bounded relative motion in the rotating frame traces the two-to-one along-track to radial ellipse of the Clohessy-Wiltshire equations.

Sources: the relative-motion (Clohessy-Wiltshire) equations and constellation geometry follow Curtis [16] and Vallado [17].

The eighty-one satellites do not fly in a rigid grid; they orbit in a loose formation governed by the linearized relative equations of motion. About a circular reference orbit of mean motion n, the Clohessy-Wiltshire (Hill) equations describe a satellite’s position relative to a co-orbiting frame:

x−3n2x−2ny=ax, y+2nx=ay, z+n2z=az.

In the absence of forcing, the radial (x) and cross-track (z) modes oscillate at the orbital rate n, while the along-track (y) motion contains a secular term: any small difference in energy or any differential acceleration produces a drift that grows linearly in time, and a differential along-track acceleration produces a drift that grows as t2. This is the central challenge of the formation: because the satellites have slightly different ballistic coefficients, differential atmospheric drag, itself varying by a factor of one hundred over the solar cycle, drives an along-track drift of order 1.5 δadrag t2. With a 150-m spacing budget, even a small differential acceleration consumes the margin within hours to days, so the formation requires active relative control. The propellant for this control is modest (Table T7), but the sensing and autonomy are not: maintaining a sub-200-m formation while remaining able to perform a collision-avoidance maneuver, without that maneuver endangering a neighbor, is an unsolved control problem at this density, and it is the operational face of the formation paradox.

Derivation of the relative equations of motion. The Hill (Clohessy-Wiltshire) relations of Eq. (28.1) follow from linearizing the two-body acceleration about a circular reference orbit. Let the chief move on a circular orbit of radius r0 with mean motion n=μ/r03, and write the deputy position as r0+x in the radial direction within a frame rotating at constant rate n. In a rotating frame the absolute acceleration carries Coriolis and centrifugal terms,

aabs=rrel+2 n×rrel+n×(n×rrel),

with n=n z. Expanding the gravitational acceleration to first order in the small displacement, the radial gradient gives −μ/(r0+x)2≈−n2r0+2n2x, in which the constant term is cancelled by the centrifugal term n2r0 acting on the reference radius. Collecting the radial, along-track, and cross-track components reproduces Eq. (28.1).

Closed-form drift and the along-track secular term. Setting the forcing to zero and solving the homogeneous system shows that the along-track coordinate accumulates a term proportional to the initial radial offset and any energy mismatch. For a constant differential acceleration δa applied along-track, integrating y+2nx=δa together with the radial equation yields, after the bounded oscillatory parts are removed, the secular growth

ysec(t)≈32 δa t2,

which is the origin of the 1.5 δadrag t2 estimate quoted in the chapter. The drift therefore scales quadratically in time and linearly in the differential acceleration.

Worked estimate. Take a representative differential drag acceleration δadrag=1×10−7 m s−2 and ask when the along-track offset reaches the 150-m spacing budget. Inverting Eq. (28.3),

tbudget=2 Δymax3 δadrag=2(150)3(10−7) s≈3.2×104 s,

about nine hours, consistent with the chapter’s statement that margin is consumed within hours to days.

Design implication. Because uncorrected along-track error grows as t2 while the control budget is fixed at the 150-m spacing, the formation must close its relative-navigation and thrust loop on a timescale well inside tbudget; this sets the required cadence of differential drag estimation and of the autonomous station-keeping maneuvers rather than the total propellant.

## 29. Comparison with Terrestrial Data Centers

Abstract. This chapter sets the orbital architecture against terrestrial hyperscale data centers, dimension by dimension, to locate where each genuinely differs.

It is instructive to place the orbital architecture beside a terrestrial hyperscale facility on the metrics that matter (Table T13). Orbit wins decisively on cooling overhead and water, and it offers a continuous solar resource and freedom from land and grid constraints. It loses decisively on serviceability (no repair), on the cost and risk of launch, on radiation-induced throughput loss, and on the debris and disposal obligations that have no terrestrial analogue. The reading is that the two are not substitutes today: orbit is competitive only for workloads that genuinely value what it uniquely provides, and only once launch economics enter the parity band of Chapter 22.

Table T13, Orbital versus terrestrial data center (qualitative).

| Dimension | Terrestrial hyperscale | Orbital (this study) |
| --- | --- | --- |
| Cooling | chillers/water, PUE ~1.1-1.6 | radiative to 2.7 K; no water |
| Water use | millions of liters/yr | none |
| Power | grid, intermittent renewables | continuous solar (dawn-dusk) |
| Land | large footprint | none |
| Serviceability | hot-swap, on-site repair | none (no repair) |
| Radiation | negligible | ~10-20 % throughput tax |
| Disposal | decommission in place | mandatory propulsive de-orbit |
| Debris risk | none | binding constraint |
| Launch / deployment | trucking | USD/kg launch, dominant early cost |
| Latency (long-haul) | fiber (n=1.47) | vacuum mesh wins beyond ~3,000 km |

Power usage effectiveness as the cooling-overhead metric. The terrestrial column of Table T13 quotes a power usage effectiveness, or PUE, in the band 1.1 to 1.6. This single figure of merit is defined as the ratio of total facility power drawn from the grid to the power that actually reaches the computing hardware,

PUE=PfacilityPIT=PIT+Pcool+PauxPIT=1+Pcool+PauxPIT,

where Pcool is the power consumed by chillers, pumps, and fans, and Paux collects lighting, power-conversion losses, and other ancillaries. The fraction of delivered power that performs computation is the reciprocal, ηinfra=1/PUE. For a terrestrial facility at PUE=1.5 this useful fraction is 1/1.5=0.67, so one third of the purchased energy is spent moving heat rather than computing. The orbital architecture rejects heat by passive radiation to the 2.7 K sky, as derived in Chapter 10, so its dedicated cooling term Pcool collapses toward the pumping power of the internal coolant loop alone; this is the precise sense in which orbit wins on cooling overhead.

Radiation throughput tax. The 10 to 20 percent figure in the radiation row is an effective availability reduction. If single-event upsets and dose-driven derating remove a fraction τr of useful cycles, the delivered compute of equation (17.1) is scaled to

Ceff=n Ppeak MFU ar (1−τr),

so that the parity comparison must be made on Ceff, not on nameplate capacity. With τr in the stated range the orbital cluster must over-provision hardware by a factor 1/(1−τr) to match a terrestrial throughput target.

Latency crossover. The latency row asserts that a vacuum mesh overtakes terrestrial fiber beyond roughly 3,000 km. Signal speed in fiber is c/n with refractive index n=1.47, while a free-space optical link propagates at c. For a great-circle ground separation L the one-way times are tfiber=nL/c and, neglecting the modest path stretch to orbit, tspace≈L/c, so the vacuum advantage grows linearly with distance,

Δt=tfiber−tspace=(n−1) Lc.

At L=3,000 km this gives Δt=0.47×3×106/(3×108)≈4.7 ms of saved one-way latency before any orbital path penalty is charged. The design implication is that orbit competes on latency only for genuinely intercontinental traffic, and on cost only inside the launch-parity band of Chapter 22; for regional workloads the terrestrial facility remains the lower-risk choice.

# Back matter

## References

- Agüera y Arcas et al. (2025). Towards a future space-based, highly scalable AI infrastructure system design. arXiv:2511.19468.

- Google. Project Suncatcher. blog.google/innovation-and-ai/technology/research/google-project-suncatcher/

- Planet Labs. Platform for Project Suncatcher. planet.com/pulse/

- NVIDIA. Starcloud, H100 in orbit. blogs.nvidia.com/blog/starcloud/

- Axiom Space. Orbital Data Center nodes. axiomspace.com

- Liang et al. (2022). Link Budget Analysis for Free-Space Optical Satellite Networks. arXiv:2204.13177.

- FCC. Five-Year Rule (FCC 22-74). fcc.gov ; Federal Register 2024-17093.

- NASA. ORDEM 3.2 User Guide. NTRS 20230014989.

- ESA. MASTER / DRAMA debris tools. esa.int/Space_Safety/Space_Debris/

- SPENVIS. SHIELDOSE. spenvis.oma.be

- Picone et al. (2002). NRLMSISE-00 empirical atmosphere model. JGR, doi:10.1029/2002JA009430.

- NVIDIA H100 / B200 / GB200 datasheets; Google Cloud TPU v6e documentation; Azur/Spectrolab solar-cell datasheets; NASA ISS Active Thermal Control System overview.

- NASA Systems Engineering Handbook, NASA/SP-2016-6105.

- AIAA S-120-2006, Mass Properties Control for Space Systems.

- NASA Goddard Integrated Design Center, concurrent-design margin practice (NTRS 20120013284).

- Curtis, H. D. Orbital Mechanics for Engineering Students, 4th ed. Butterworth-Heinemann, 2020.

- Vallado, D. A. Fundamentals of Astrodynamics and Applications, 4th ed. Microcosm Press, 2013.

- Wertz, J. R., Everett, D. F., Puschell, J. J. (eds.). Space Mission Engineering: The New SMAD. Microcosm Press, 2011.

- Larson, W. J., Wertz, J. R. (eds.). Space Mission Analysis and Design, 3rd ed. Microcosm/Kluwer, 1999.

- Gilmore, D. G. (ed.). Spacecraft Thermal Control Handbook, Volume I: Fundamental Technologies, 2nd ed. The Aerospace Press, 2002.

- Incropera, F. P., DeWitt, D. P., Bergman, T. L., Lavine, A. S. Fundamentals of Heat and Mass Transfer, 7th ed. Wiley, 2011.

- Tribble, A. C. The Space Environment: Implications for Spacecraft Design, rev. ed. Princeton University Press, 2003.

- Pisacane, V. L. (ed.). The Space Environment and Its Effects on Space Systems, 2nd ed. AIAA Education Series, 2016.

- Hastings, D., Garrett, H. Spacecraft-Environment Interactions. Cambridge University Press, 1996.

- Vette, J. I. The AE-8 Trapped Electron Model Environment (NSSDC/WDC-A-R&S 91-24); Sawyer, D. M., Vette, J. I. AP-8 Trapped Proton Environment (NSSDC/WDC-A-R&S 76-06). NASA GSFC.

- Ginet, G. P. et al. AE9, AP9 and SPM: New Models for Specifying the Trapped Energetic Particle and Space Plasma Environment. Space Science Reviews 179, 2013.

- Seltzer, S. M. SHIELDOSE: A Computer Code for Space-Shielding Radiation Dose Calculations. NBS Technical Note 1116, 1980.

- Kessler, D. J., Cour-Palais, B. G. Collision Frequency of Artificial Satellites: The Creation of a Debris Belt. Journal of Geophysical Research 83(A6), 1978.

- Klinkrad, H. Space Debris: Models and Risk Analysis. Springer-Praxis, 2006.

- Hemmati, H. (ed.). Deep Space Optical Communications. JPL Deep-Space Communications and Navigation Series, Wiley, 2006.

- Sutton, G. P., Biblarz, O. Rocket Propulsion Elements, 9th ed. Wiley, 2017.

- Wertz, J. R. (ed.). Spacecraft Attitude Determination and Control. D. Reidel, 1978.

- Sidi, M. J. Spacecraft Dynamics and Control: A Practical Engineering Approach. Cambridge University Press, 1997.

- ECSS. Space Engineering: Thermal Control General Requirements (ECSS-E-ST-31C) and Space Environment (ECSS-E-ST-10-04C). European Cooperation for Space Standardization.

- O’Hanlon, M. et al.; SpaceX published launch-manifest and reusability data; FAA commercial-space launch records, for the launch-cadence and cost-trajectory inputs of Chapter 22.

## Appendix A, Master List of Governing Equations

| # | Quantity | Equation |
| --- | --- | --- |
| 4.1 | Circular speed | v=μ/a |
| 4.2 | Period / mean motion | T=2πa3/μ, n=μ/a3 |
| 4.3 | Specific energy | E=−μ/2a |
| 5.2 | SSO inclination | cosi=−2Ωsuna2/(3nJ2RE2) |
| 6.1 | Critical beta | β*=arcsin(RE/(RE+h)) |
| 7.3 | Orbital decay | a=−CD(A/m) ρ μa |
| 8.1 | Dose-depth | D(x)=D0e−x/λ+D∞ |
| 9.1 | Debris risk | P=1−e−ΦANt |
| 10.2 | Stefan-Boltzmann | Eb=σT4 |
| 10.3 | Radiator area | Arad=Q/[εσT4ns(1−fp)] |
| 11.1 | Array power | P=S0A η κ ηpath |
| 12.1 | Panel frequency | f1=(β/2π)D/(ρAa4) |
| 13.1 | Gravity-gradient torque | τgg=32 n2 ΔI sin2θ |
| 14.1 | Telescope gain / FSPL | G=(πD/λ)2, Lfs=(4πR/λ)2 |
| 15.1 | De-orbit Δv | Δv=vc1(1−2r2/(r1+r2)) |
| 15.2 | Propellant | mp=mdry(eΔv/Ispg0−1) |
| 16.1 | Reliability | R(t)=e−t/MTBF |
| 16.2 | k-of-n | Rsys=i=knnipi(1−p)n−i |
| 17.1 | Delivered compute | nPpeak MFU ar |
| 21.1 | LCOE | (CRF⋅CAPEX+OPEX)/useful PFLOP-hr |

## Appendix B, Nomenclature

See the Nomenclature table in the front matter; every symbol used in the equations above is defined there with its units.

## Appendix C, Data Sources and Confidence

Every model input is tabulated with its value, source, and a confidence flag (official / model / estimate) in the companion repository’s systems-model/docs/SOURCES.md. The dominant caveats are repeated here: the chip power is undisclosed and treated as an estimate; the solar-cycle density swing of roughly one hundred is the largest physical uncertainty and is shown as a band; and the 1-mm-to-1-cm debris flux is the least model-validated quantity.

## Appendix D, Reproduction and Code

The entire analysis is open. From the repository,

git clone https://github.com/Samarjithbiswas/space-based-ai-datacenter
cd space-based-ai-datacenter && pip install numpy matplotlib pytest
python reproduce_all.py        # both models, all figures, equation images
python report/predictions.py   # the launch-cost-parity Monte-Carlo

The systems model comprises seventeen subsystem modules plus an integration layer and a Monte-Carlo engine, validated by 108 automated tests. Each module’s docstring states its governing equation; the mathematical derivations in this volume are the authoritative reference. Independent reproduction, extension, and correction are welcome through the repository’s issues and pull requests.

For completeness, the core physics functions are listed below; each is a direct, pure implementation of the corresponding equation in this volume.

Orbital decay, Eq. (7.3).

def lifetime_years(alt_km, area_to_mass, scenario="mod", cd=2.2):
    a, t, yr, dt = R_E + alt_km*1e3, 0.0, 3.15576e7, 3600*6
    while a > R_E + 120e3 and t < 2000*yr:
        h = (a - R_E)/1e3
        a += -cd*area_to_mass*density(h, scenario)*np.sqrt(MU*a)*dt; t += dt
        dt = 3600*48 if (a-R_E) > 500e3 else (3600*12 if (a-R_E) > 300e3 else 3600*2)
    return t/yr

Radiator sizing, Eq. (10.3).

def radiator_area(heat_W, temp_C, emissivity=0.90, sides=2, parasitic=0.12):
    T = temp_C + 273.15
    return heat_W / (emissivity * SIGMA * T**4 * sides * (1 - parasitic))

Debris collision probability, Eq. (9.1).

def collision_probability(cross_section_m2, n_sats, years, size="1cm", band="mid"):
    flux = debris_flux(size, band)
    return 1 - np.exp(-flux * cross_section_m2 * n_sats * years)

Controlled de-orbit Δv, Eq. (15.1).

def deorbit_delta_v(alt_km, perigee_km=180.0):
    r1, r2 = R_E + alt_km*1e3, R_E + perigee_km*1e3
    v1 = np.sqrt(MU / r1)
    return v1 * (1 - np.sqrt(2*r2 / (r1 + r2)))

Coupled thermal-throttle workload (excerpt), Chapter 17.

def throttle_factor(T_j, T_throttle=110.0, T_max=125.0):
    return float(np.clip((T_max - T_j) / (T_max - T_throttle), 0.0, 1.0))
# in the time loop: T_j = T_rad_C + P_full*r_th; u_eff = u_cmd * throttle_factor(T_j)
# C dT/dt = Q_in - eps*sigma*A_eff*T^4   (forward Euler over one orbit)

Levelized cost of compute, Eq. (21.1).

def lcoe_per_pflop_hr(capex, opex_yr, delivered_pflops, rate=0.10, life=5, util=0.8):
    crf = rate*(1+rate)**life / ((1+rate)**life - 1)
    return (crf*capex + opex_yr) / (delivered_pflops * 8766.0 * util)

## Appendix E, Extended Derivations

This appendix gives the full step-by-step derivations of the results quoted in compressed form in the main text, so that the volume is mathematically self-contained.

### E.1 Stefan-Boltzmann law from Planck’s law

Starting from the spectral radiance (10.1), the hemispherical emissive power is the integral of the radiance over the emitting hemisphere and all wavelengths. The angular integral over the hemisphere, weighted by the projection factor cosθ, is

hemi​cosθ dΩ=02π​​0π/2cosθ sinθ dθ dϕ=2π⋅12=π.

Hence Eb(T)=π0∞B(λ,T) dλ. Substituting Planck’s law and changing variables to u=hPc/(λkBT), so that λ=hPc/(ukBT) and dλ=−hPc/(u2kBT) du, the wavelength integral becomes

Eb(T)=2πkB4T4hP3c20∞u3eu−1 du.

The remaining integral is a standard result, 0∞u3/(eu−1) du=π4/15, giving

Eb(T)=2π5kB415 hP3c2 T4≡σT4,

which both proves the fourth-power law and yields the Stefan-Boltzmann constant in terms of fundamental constants. This is the foundation of every radiator result in Chapter 10.

### E.2 The Poisson impact model from the binomial limit

Divide the mission duration t into n equal intervals Δt=t/n. In each interval the probability of one impact is p=ΦAΔt (flux times cross-section times time), and impacts in different intervals are independent. The number of impacts K is therefore binomial, P(K=k)=nkpk(1−p)n−k. Hold the expected count Λ=np=ΦAt fixed and let n→∞ (so p→0). Then nkpk→Λk/k! and (1−p)n−k→e−Λ, so

P(K=k)=Λke−Λk!.

The probability of at least one impact is P(K≥1)=1−P(K=0)=1−e−Λ. For N independent satellites the expected count scales to Λ=ΦANt, giving equation (9.1). This is exact in the limit of many small independent intervals, the appropriate regime for rare debris impacts.

### E.3 The Hohmann de-orbit impulse from vis-viva

The vis-viva relation for any Keplerian orbit is v2=μ(2/r−1/a), obtained from conservation of the specific energy E=v2/2−μ/r=−μ/2a. Consider a transfer ellipse whose apogee touches the circular orbit at r1 and whose perigee is lowered to r2; its semi-major axis is at=(r1+r2)/2. The speed required at the apogee of this transfer is

vapo=μ2r1−1at=μ2r1−2r1+r2=μr12r2r1+r2.

The satellite is initially on the circular orbit at speed vc1=μ/r1, so the retrograde impulse needed is the difference,

Δvdeorbit=vc1−vapo=vc11−2r2r1+r2,

which is equation (15.1). Lowering the perigee into the dense atmosphere (here to 180 km) guarantees re-entry within one or two orbits, satisfying the disposal requirement.

### E.4 The Clohessy-Wiltshire relative equations

Let a chief satellite move on a circular orbit of radius r0 and mean motion n=μ/r03, and let a deputy be displaced by a small vector (x,y,z) in the rotating Hill frame (radial, along-track, cross-track). Writing Newton’s equation for the deputy, expanding the gravity gradient to first order in the small displacement, and accounting for the frame rotation (Coriolis and centrifugal terms) yields the linearized system

x−2ny−3n2x=ax, y+2nx=ay, z+n2z=az.

The cross-track equation is a simple harmonic oscillator at frequency n. The in-plane equations couple radial and along-track motion; their unforced solution contains bounded oscillations at n plus a secular along-track drift proportional to any initial energy mismatch. Crucially, a constant differential along-track acceleration ay (such as differential drag) integrates twice into an along-track displacement that grows as 12ayt2 to leading order, augmented by the Coriolis coupling to a coefficient of order 32, which is the quadratic-in-time drift discussed in Chapter 28 and the reason the formation needs active relative control.

### E.5 The rocket equation

A rocket of instantaneous mass m expelling propellant at exhaust speed ve relative to the vehicle changes its momentum according to m dv=−ve dm (the mass decreases as propellant is ejected, hence the minus sign). Separating variables and integrating from the initial mass m0 to the final (dry) mass mf,

0Δvdv=−vem0mfdmm ⇒ Δv=velnm0mf.

With ve=Ispg0 and mf=mdry, the propellant mass mp=m0−mf follows by inverting:

mp=mdryeΔv/(Ispg0)−1,

which is equation (15.2). The exponential dependence is why a high specific impulse (electric propulsion) so dramatically reduces propellant mass for a given Δv.

### E.6 k-of-n redundancy

If n identical units each survive independently with probability p, the number that survive is binomial. The system requires at least k survivors, so its reliability is the upper tail of the binomial distribution,

Rsys=i=knni pi(1−p)n−i.

Two special cases are familiar: a parallel (one-of-n) system has R=1−(1−p)n, and a series (n-of-n) system has R=pn. This expression sizes both the spare accelerators within a satellite and the over-provisioned satellites within a cluster (Chapter 16).

### E.7 Newton-Raphson for the radiator quartic

To find the radiator temperature at fixed area, one solves f(T)=Q−εσAT4=0. Newton’s method linearizes f about the current iterate: f(T+δ)≈f(T)+f′(T)δ, set to zero gives δ=−f(T)/f′(T), hence

Tk+1=Tk−f(Tk)f′(Tk)=Tk+Q−εσATk44 εσATk3.

Because f is smooth and strictly decreasing for T>0 (its derivative −4εσAT3 never vanishes there), the iteration converges quadratically from any reasonable start; in practice, from T0=300 K it reaches a tolerance of 0.01 K in two to four steps.

### E.8 The capital-recovery factor

The capital-recovery factor converts a present capital sum C into an equivalent stream of n equal annual payments X at discount rate r. The present value of the payment stream is a finite geometric series,

C=t=1nX(1+r)t=X 1−(1+r)−nr.

Solving for the annual payment per unit capital gives

CRF=XC=r(1+r)n(1+r)n−1,

used in the levelized-cost-of-compute formula (21.1) to annualize the satellite hardware and launch cost over its operational life.